Cover

VarCom

Reference Manual

smartPX

ORIGINAL DOCUMENT

Manual Revision: 1.0  - Draft v6 (8 Mar.2024)

Firmware Version: 3.0.1.8

Revision History

| Doc. Rev. | Date | Remarks |
| --- | --- | --- |
| 1.0 |  | Initial release |

Copyright Notice

© 2024 Servotronix Motion Control Ltd.

All rights reserved. No part of this work may be reproduced or transmitted in any form or by any means without prior written permission of Servotronix Motion Control Ltd..

Disclaimer

This product documentation was accurate and reliable at the time of its release. Servotronix Motion Control Ltd. reserves the right to change the specifications of the product described in this manual without notice at any time.

Trademarks

All marks in this manual are the property of their respective owners.

Contact Information

Servotronix Motion Control Ltd.
21C Yagia Kapayim Street
Petach Tikva 49130 Israel
Tel: +972 (3) 927 3800
Fax: +972 (3) 922 8075

Website: www.servotronix.com

Technical Support

If you need assistance with the installation and configuration of the product, contact Servotronix technical support: tech.support@servotronix.com

Contents

1	Introduction

1.1	VarCom Overview	7

1.2	Manual Format	7

2	Parameters

2.1	Brakes	9

Brake.Engage.Time	9

Brake.Exist	9

Brake.Mode	10

Brake.Release.Time	10

Brake.Release.Timeout	11

Brake.Status	11

Brake.Status.Signal	12

2.2	Current Control	13

Curr.Autotune	13

Curr.Cmd.User	13

Curr.Db.Istop	14

Curr.Db.Kpfac	14

Curr.Dqcomp	15

Curr.Grav	15

Curr.Iff	16

Curr.Iffwoeso	16

Curr.Kbemf	17

Curr.Kd	17

Curr.Kff	18

Curr.Ki	18

Curr.Kiv	19

Curr.Kp	19

Curr.Kvg	20

Curr.Lim.Act	20

Curr.Lim.User	21

Curr.Lpf	21

Curr.Offset.Uint	22

Curr.Sensor.Offset.Ia	22

Curr.Sensor.Offset.Ic	22

Curr.Sensor.Offset.Lpf	23

Curr.Sensor.Status.A	23

Curr.Sensor.Status.C	24

Curr.Target.Uint	24

Curr.Vd	25

Curr.Vq	25

Com.Angle (to be moved to “Commutation”)	25

Com.Angle.Raw (to be moved to “Commutation”)	26

Eso.Dcomp.Inuse (to be moved to “Commutation”)	26

Eso.Iff	27

Ia	27

Iavg	27

Ic	28

Icmd	28

Icmd.Fweak.Base.Speed1	28

Icmd.Fweak.Base.Speed2	29

Icmd.Fweak.Enable	29

Icmd.Fweak.Gain	30

Icmd.Noise.Level	30

Icmd.Raw	31

Id	31

Iq	32

Vel.Curr	32

2.3	Digital Inputs/Outputs	33

Digital.In	33

Digital.In.Debounce	33

Digital.In.Map	34

Digital.In.Mode	34

Digital.In.Estop	35

Digital.In.Numof	35

Digital.In.Polarity	36

Digital.Out	36

Digital.Out.Brake	37

Digital.Out.Map	37

Digital.Out.Mask	37

Digital.Out.Mode	38

Digital.Out.Numof	38

Digital.Out.Polarity	39

Digital.Led.Color	39

2.4	Info	40

Fb.Ecat.Status	40

Fpga.Ver	41

Info	42

Status	43

Stat.Time.En	43

Stat.Time.En.Total	44

Stat.Time.On	44

Stat.Time.On.Total	44

Sys.Time.Date	45

Sys.Time.Day	45

Sys.Time.Format	46

Sys.Time.Hour	46

Sys.Time.Min	47

Sys.Time.Month	47

Sys.Time.Sec	48

Sys.Time.Timedate	48

Sys.Time.Year	49

Ver	49

2.5	Motion	50

Jog	50

Move	51

P2P.Abs	51

P2P.Acc	52

P2P.Acmd.Raw	52

P2P.Cyclic	53

P2P.Dec	53

P2P.Decstop	54

P2P.Move.Filter	54

P2P.Pcmd.Raw	55

P2P.Pos.Neg.Lim	55

P2P.Pos.Pos.Lim	56

P2P.Profile.Complete	56

P2P.Ptpvcmd.Raw	57

P2P.Vcruise	57

Step.Acc	58

Step.Dec	58

Step.Delay1	59

Step.Delay2	60

Step.Jog1	60

Step.Jog2	61

Step.On	61

Step.Time1	62

Step.Time2	62

2.6	Motor	63

Motor.Dir	63

Motor.Fric.Dynamic	63

Motor.Ipeak	64

Motor.Icont	64

Motor.Ibrake	65

Motor.J	65

Motor.Kt	66

Motor.Kf	66

Motor.Lmjr	67

Motor.L	67

Motor.Mass	68

Motor.Name	68

Motor.Phase.Mphase	69

Motor.Phase.Mvangle.Phase	69

Motor.Phase.Mvangle.Speed	70

Motor.Phase.Phasefind.Icmdint	70

Motor.Phase.Phasefind.Mode	71

Motor.Phase.Phasefind.On	71

Motor.Phase.Ver	72

Motor.Phase.Zeroing.On	72

Motor.Phase.Zeroing.State	73

Motor.Pitch	73

Motor.Poles	74

Motor.R	74

Motor.Saliency	75

Motor.Speed	75

Motor.Type	76

Motor.uwSwap	76

2.7	Recording	77

Recdone	77

Recgap	78

Recoff	78

Recoffs	79

Record	79

Recordclose	80

Rectrig	81

2.8	Temperature	82

Temp.Cpu	82

Temp.Cpu.Cor	82

Temp.Cpu.Dsp	82

Temp.Cpu.Gpu	83

Temp.Cpu.Iva	83

Temp.Cpu.Mpu	83

Temp.Ipm	84

Temp.Motor	84

Temp.Motor.Flt.High	85

Temp.Motor.Flt.Low	85

Temp.Motor.Wrn.High	86

Temp.Motor.Wrn.Low	86

2.9	Variable Gains	87

Vargains.Active	87

Vargains.Axis.Cplg	87

Vargains.Axis.Desc	88

Vargains.Axis.Offset	89

Vargains.Axis.Posfactor	89

Vargains.Axis.Status	90

Vargains.Change.Info	90

Vargains.Enable	91

Vargains.Execute	91

Vargains.Gainset.Active	92

Vargains.Gainset.Desc	93

Vargains.Gainset.Inuse	94

Vargains.Gainset.Status	94

Vargains.Inuse	95

Vargains.Joint.Pcmd	95

Vargains.Payload	96

Vargains.Pcmdact	96

Vargains.Status	97

Vargains.Status.Cntr	97

2.10	Velocity Control	98

Vact	98

Vact.Execute	98

Vact.Kalman.Acqr	99

Vact.Kalman.Design	99

Vact.Kalman.Design.Status	100

Vact.Kalman.Pobs	100

Vact.Kalman.Q	101

Vact.Kalman.R	101

Vact.Kpafrv.Factor	102

Vcmd	102

Vel.Act	103

Vel.Ctrl.Mode	103

Vel.Ctrl.Mode.Act	104

Vel.Err	104

Vel.Err.Max	105

Vel.Execute	105

Vel.Fb	106

Vel.Fb.Filter	106

Vel.Fb.Filter.Act	107

Vel.Fb.Filter.Current	107

Vel.Filt1.Lead.F	108

Vel.Filt1.Lead.F.Act	108

Vel.Filt1.Lead.Phase	109

Vel.Filt1.Lead.Phase.Act	109

Vel.Filt1.Mode	110

Vel.Filt1.Mode.Act	111

Vel.Filt1.Notch.Bw	111

Vel.Filt1.Notch.Bw.Act	112

Vel.Filt1.Notch.Center	112

Vel.Filt1.Notch.Center.Act	113

Vel.Filt1.Notch.Phase	113

Vel.Filt1.Notch.Phase.Act	114

Vel.Filt1.Pole.F	114

Vel.Filt1.Pole.F.Act	115

Vel.Filt1.User	115

Vel.Filt1.Xpole.F	116

Vel.Filt1.Xpole.F.Act	116

Vel.Filt1.Xpole.Xi	117

Vel.Filt1.Xpole.Xi.Act	117

Vel.Filt1.Xzero.F	118

Vel.Filt1.Xzero.F.Act	118

Vel.Filt1.Xzero.Xi	119

Vel.Filt1.Xzero.Xi.Act	119

Vel.Filt1.Zero.F	120

Vel.Filt1.Zero.F.Act	120

Vel.Filt2.Lead.F	121

Vel.Filt2.Lead.F.Act	121

Vel.Filt2.Lead.Phase	122

Vel.Filt2.Lead.Phase.Act	122

Vel.Filt2.Mode	123

Vel.Filt2.Mode.Act	124

Vel.Filt2.Notch.Bw	124

Vel.Filt2.Notch.Bw.Act	125

Vel.Filt2.Notch.Center	125

Vel.Filt2.Notch.Center.Act	126

Vel.Filt2.Notch.Phase	126

Vel.Filt2.Notch.Phase.Act	127

Vel.Filt2.Pole.F	127

Vel.Filt2.Pole.F.Act	128

Vel.Filt2.User	128

Vel.Filt2.Xpole.F	129

Vel.Filt2.Xpole.F.Act	129

Vel.Filt2.Xpole.Xi	130

Vel.Filt2.Xpole.Xi.Act	130

Vel.Filt2.Xzero.F	131

Vel.Filt2.Xzero.F.Act	131

Vel.Filt2.Xzero.Xi	132

Vel.Filt2.Xzero.Xi.Act	132

Vel.Filt2.Zero.F	133

Vel.Filt2.Zero.F.Act	133

Vel.Filt3.Lead.F	134

Vel.Filt3.Lead.F.Act	134

Vel.Filt3.Lead.Phase	135

Vel.Filt3.Lead.Phase.Act	135

Vel.Filt3.Mode	136

Vel.Filt3.Mode.Act	137

Vel.Filt3.Notch.Bw	137

Vel.Filt3.Notch.Bw.Act	138

Vel.Filt3.Notch.Center	138

Vel.Filt3.Notch.Center.Act	139

Vel.Filt3.Notch.Phase	139

Vel.Filt3.Notch.Phase.Act	140

Vel.Filt3.Pole.F	140

Vel.Filt3.Pole.F.Act	141

Vel.Filt3.User	141

Vel.Filt3.Xpole.F	142

Vel.Filt3.Xpole.F.Act	142

Vel.Filt3.Xpole.Xi	143

Vel.Filt3.Xpole.Xi.Act	143

Vel.Filt3.Xzero.F	144

Vel.Filt3.Xzero.F.Act	144

Vel.Filt3.Xzero.Xi	145

Vel.Filt3.Xzero.Xi.Act	145

Vel.Filt3.Zero.F	146

Vel.Filt3.Zero.F.Act	146

Vel.Filt4.Lead.F	147

Vel.Filt4.Lead.F.Act	147

Vel.Filt4.Lead.Phase	148

Vel.Filt4.Lead.Phase.Act	148

Vel.Filt4.Mode	149

Vel.Filt4.Mode.Act	150

Vel.Filt4.Notch.Bw	150

Vel.Filt4.Notch.Bw.Act	151

Vel.Filt4.Notch.Center	151

Vel.Filt4.Notch.Center.Act	152

Vel.Filt4.Notch.Phase	152

Vel.Filt4.Notch.Phase.Act	153

Vel.Filt4.Pole.F	153

Vel.Filt4.Pole.F.Act	154

Vel.Filt4.User	154

Vel.Filt4.Xpole.F	155

Vel.Filt4.Xpole.F.Act	155

Vel.Filt4.Xpole.Xi	156

Vel.Filt4.Xpole.Xi.Act	156

Vel.Filt4.Xzero.F	157

Vel.Filt4.Xzero.F.Act	157

Vel.Filt4.Xzero.Xi	158

Vel.Filt4.Xzero.Xi.Act	158

Vel.Filt4.Zero.F	159

Vel.Filt4.Zero.F.Act	159

Vel.Integrator	160

Vel.Integrator.Ff	160

Vel.Kp.Vg	161

Vel.Lim.Act	161

Vel.Neglim	162

Vel.Pdff.Kfr	162

Vel.Pdff.Kfr.Act	163

Vel.Pdff.Ki	164

Vel.Pdff.Ki.Act	164

Vel.Pdff.Kp	165

Vel.Pdff.Kp.Act	165

Vel.Poslim	166

Vel.Pp.Bw	166

Vel.Pp.Bw.Act	167

Vel.Pp.Gain	167

Vel.Pp.Gain.Act	168

Vel.Pp.Tf	168

Vel.Pp.Tf.Act	169

Vel.Prefilt.Lead.F	169

Vel.Prefilt.Lead.F.Act	170

Vel.Prefilt.Lead.Phase	170

Vel.Prefilt.Lead.Phase.Act	171

Vel.Prefilt.Mode	171

Vel.Prefilt.Mode.Act	172

Vel.Prefilt.Notch.Bw	172

Vel.Prefilt.Notch.Bw.Act	173

Vel.Prefilt.Notch.Center	173

Vel.Prefilt.Notch.Center.Act	174

Vel.Prefilt.Notch.Phase	174

Vel.Prefilt.Notch.Phase.Act	175

Vel.Prefilt.Pole.F	175

Vel.Prefilt.Pole.F.Act	176

Vel.Prefilt.User	176

Vel.Prefilt.Xpole.F	177

Vel.Prefilt.Xpole.F.Act	177

Vel.Prefilt.Xpole.Xi	178

Vel.Prefilt.Xpole.Xi.Act	178

Vel.Prefilt.Xzero.F	179

Vel.Prefilt.Xzero.F.Act	179

Vel.Prefilt.Xzero.Xi	180

Vel.Prefilt.Xzero.Xi.Act	180

Vel.Prefilt.Zero.F	181

Vel.Prefilt.Zero.F.Act	181

Vlim	182

2.11	Warnings and Faults	183

Flt.Action	183

Flt.Active.List	184

Flt.Can.Error.Code	184

Flt.Clear	185

Flt.Unmute	185

Flt.Exist	186

Flt.Hist	186

Flt.Last	187

Inhibit.Active.List	187

Inhibit.Exist	188

Inhibit.Hist	188

Wrn.Active.List	188

Wrn.Exist	189

Wrn.Hist	189

# Introduction

## VarCom Overview

When the host and drive are communicating over a serial connection, a proprietary set of parameters (variables) and commands, called VarCom, are used to configure, control and monitor the drive.

Some parameters are read-only or write-only, while others have both read and write access. Parameters can be stored in the smartPX’s non-volatile flash memory for use at each power-up.

## Manual Format

This manual details the entire set of VarCom parameters.

Parameters and commands are presented as follows:

| Parameter Name | The name of the parameter or the command, used in program code. For example: DIGITAL.IN.NUMOF |
| --- | --- |
| Definition | A short descriptive name of the parameter or command. For example: Number of digital inputs |
| Type | Parameter (R/W): A read/write parameter. Parameter (R): A read-only parameter. Parameter (W): A write-only parameter. Command (not yet in documentation) Bus (R): Global parameter that is shared internally among modules in system |
| Description | A description of the parameter or command. |
| Syntax | [ ] Square brackets are part of the parameter/command name, and delineate an axis number or an index (of an array parameter). For example: FLT.UNMUTE[axis] [0] Explain … For example: DIGITAL.OUT.MODE[0][index] = <value> ? When a question mark is used before a parameter name, it serves to query the system for information. For example: ?BRAKE.EXIST[axis] Italicized text represents a value or argument that is specified when the command code is entered. For example: (Lisa’s suggestions) arg; argument (usually a string or variable). axis; axis. time; duration. num; number. oper; operator (such as <, >, =). pos; position. prog; program. pvect; position vector. value; value. |
| Firmware | The earliest version, or specific versions, in which the described functionality is available. |
| Range | Discrete values or ranges of values. |
| Value Range | explain |
| Index Range | explain |
| Default Value | The parameter’s default (factory-defined) value. |
| Units | When a parameter value represents a unit of measure, the type of unit is specified. |
| CAN Index | If relevant, the index of the corresponding CANopen object. |

# Parameters

## Brakes

### Brake.Engage.Time

| Parameter Name | BRAKE.ENGAGE.TIME |
| --- | --- |
| Definition | Motor brake engage time |
| Type | Parameter (R/W) |
| Description | Defines/reports the time the brake must be engaged before actual disable occurs. |
| Syntax | Write: BRAKE.ENGAGE.TIME[axis] = <value> Read: ?BRAKE.ENGAGE.TIME[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 10000 |
| Default Value | 100 |
| Units | ms |
| CAN Index | NA |

### Brake.Exist

| Parameter Name | BRAKE.EXIST |
| --- | --- |
| Definition | Motor brake exist |
| Type | Parameter (R/W) |
| Description | Defines/reports whether motor has brake functionality. 0 = No motor brake 1 = Motor brake |
| Syntax | Write: BRAKE.EXIST[axis] = <value> Read: ?BRAKE.EXIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Brake.Mode

| Parameter Name | BRAKE.MODE |
| --- | --- |
| Definition | Brake mode |
| Type | Parameter (R/W) |
| Description | Defines the motor brake behavior mode: 0 = Single axis mode 1 = Multi-axis mode (single axis can not be enabled) 2 = Brake test mode (internal use) |
| Syntax | Write: BRAKE.MODE[axis] = <value> Read: ?BRAKE.MODE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 2 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x23b3, sub-index 0 |

### Brake.Release.Time

| Parameter Name | BRAKE.RELEASE.TIME |
| --- | --- |
| Definition | Brake release time |
| Type | Parameter (R/W) |
| Description | Defines/reports the time the brake must be released before actual enable occurs. |
| Syntax | Write: BRAKE.RELEASE.TIME[axis] = <value> Read: ?BRAKE.RELEASE.TIME[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 10000 |
| Default Value | 100 |
| Units | ms |
| CAN Index | NA |

### Brake.Release.Timeout

| Parameter Name | BRAKE.RELEASE.TIMEOUT |
| --- | --- |
| Definition | Brake release timeout |
| Type | Parameter (R/W) |
| Description | Defines/reports the release brake timeout. If the brake is not released during the defined time, a warning is issued: Brake release taking too long. |
| Syntax | Write: BRAKE.RELEASE.TIMEOUT[axis] = <value> Read: ?BRAKE.RELEASE.TIMEOUT[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 10000 |
| Default Value | 1000 |
| Units | ms |
| CAN Index | NA |

### Brake.Status

| Parameter Name | BRAKE.STATUS |
| --- | --- |
| Definition | Brake released/engaged status |
| Type | Parameter (R) |
| Description | Reports the brake released/engaged status. 0 = Brake engaged 1 = Brake released |
| Syntax | Read: ?BRAKE.STATUS[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Brake.Status.Signal

| Parameter Name | BRAKE.STATUS.SIGNAL |
| --- | --- |
| Definition | Brake output signal |
| Type | Parameter (R) |
| Description | Brake.Status changes to this value after Brake.Release.Time or Brake.Engage.Time has elapsed. |
| Syntax | Read: ?BRAKE.STATUS.SIGNAL[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 255 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

## Current Control

### Curr.Autotune

| Parameter Name | CURR.AUTOTUNE |
| --- | --- |
| Definition | Gravity compensation term |
| Type | Parameter (R/W) |
| Description | Gravity compensation term |
| Syntax | Write: CURR.AUTOTUNE[axis] = <value> Read: ?CURR.AUTOTUNE[axis] |
| Firmware | 3.0.1.8 |
| Range | -1000 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | NA |

### Curr.Cmd.User

| Parameter Name | CURR.CMD.USER |
| --- | --- |
| Definition | User current command |
| Type | Parameter (R/W) |
| Description | Defines/reports the current command in Serial Current mode (-10). |
| Syntax | Write: CURR.CMD.USER[axis] = <value> Read: ?CURR.CMD.USER[axis] |
| Firmware | 3.0.1.8 |
| Range | -1000 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | 0x230e, sub-index 0 |

### Curr.Db.Istop

| Parameter Name | CURR.DB.ISTOP |
| --- | --- |
| Definition | Dynamic brake stop current |
| Type | Parameter (R/W) |
| Description | Defines/reports the maximum current allowed during the dynamic braking process. |
| Syntax | Write: CURR.DB.ISTOP[axis] = <value> Read: ?CURR.DB.ISTOP[axis] |
| Firmware | 3.0.1.8 |
| Range | 0.1 to 1000 |
| Default Value | 1000 |
| Units | NA |
| CAN Index | 0x2309, sub-index 0 |

### Curr.Db.Kpfac

| Parameter Name | CURR.DB.KPFAC |
| --- | --- |
| Definition | Dynamic Brake Gain Factor |
| Type | Parameter (R/W) |
| Description | Defines/reports the dynamic brake gain factor. |
| Syntax | Write: CURR.DB.KPFAC[axis] = <value> Read: ?CURR.DB.KPFAC[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x230a, sub-index 0 |

### Curr.Dqcomp

| Parameter Name | CURR.DQCOMP |
| --- | --- |
| Definition | Current loop cross compensation gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop cross compensation. |
| Syntax | Write: CURR.DQCOMP[axis] = <value> Read: ?CURR.DQCOMP[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2004, sub-index 0 |

### Curr.Grav

| Parameter Name | CURR.GRAV |
| --- | --- |
| Definition | Gravity compensation |
| Type | Parameter (R/W) |
| Description | Defines/reports the value of the gravity compensation constant. CURR.GRAV is added to the current loop command to compensate for gravity or a similar constant interference. |
| Syntax | Write: CURR.GRAV[axis] = <value> Read: ?CURR.GRAV[axis] |
| Firmware | 3.0.1.8 |
| Range | -1000 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | 0x2313, sub-index 0 |

### Curr.Iff

| Parameter Name | CURR.IFF |
| --- | --- |
| Definition | Actual current feedforward |
| Type | Bus (R) |
| Description | Reports the actual current feedforward. |
| Syntax | Read: ?CURR.IFF[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Curr.Iffwoeso

| Parameter Name | CURR.IFFWOESO |
| --- | --- |
| Definition | Actual current feedforward without ESO |
| Type | Bus (R) |
| Description | Reports the actual current feedforward without ESO (extended state observer?) |
| Syntax | Read: ?CURR.IFFWOESO[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Curr.Kbemf

| Parameter Name | CURR.KBEMF |
| --- | --- |
| Definition | Current loop back EMF compensation gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop back EMF compensation ratio for the current control. |
| Syntax | Write: CURR.KBEMF[axis] = <value> Read: ?CURR.KBEMF[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2003, sub-index 0 |

### Curr.Kd

| Parameter Name | CURR.KD |
| --- | --- |
| Definition | Current loop dead time compensation gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the minimum current level to start compensation for dead-time effect. |
| Syntax | Write: CURR.KD[axis] = <value> Read: ?CURR.KD[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 100 |
| Default Value | 2 |
| Units | NA |
| CAN Index | 0x20ef, sub-index 0 |

### Curr.Kff

| Parameter Name | CURR.KFF |
| --- | --- |
| Definition | Current loop feedforward gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop feedforward gain. |
| Syntax | Write: CURR.KFF[axis] = <value> Read: ?CURR.KFF[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2082, sub-index 0 |

### Curr.Ki

| Parameter Name | CURR.KI |
| --- | --- |
| Definition | Current loop integral gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop integral gain. |
| Syntax | Write: CURR.KI[axis] = <value> Read: ?CURR.KI[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2006, sub-index 0 |

### Curr.Kiv

| Parameter Name | CURR.KIV |
| --- | --- |
| Definition | Current loop derivative-integral gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop derivative-integral gain. |
| Syntax | Write: CURR.KIV[axis] = <value> Read: ?CURR.KIV[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2306, sub-index 0 |

### Curr.Kp

| Parameter Name | CURR.KP |
| --- | --- |
| Definition | Current loop proportional gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop proportional gain. |
| Syntax | Write: CURR.KP[axis] = <value> Read: ?CURR.KP[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2007, sub-index 0 |

### Curr.Kvg

| Parameter Name | CURR.KVG |
| --- | --- |
| Definition | Current loop variable gain |
| Type | Parameter (R/W) |
| Description | Defines/reports the current loop variable gain. |
| Syntax | Write: CURR.KVG[axis] = <value> Read: ?CURR.KVG[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 2 |
| Default Value | 2 |
| Units | NA |
| CAN Index | 0x2307, sub-index 0 |

### Curr.Lim.Act

| Parameter Name | CURR.LIM.ACT |
| --- | --- |
| Definition | Drive actual current limit |
| Type | Parameter (R) |
| Description | Reports the actual current limit. |
| Syntax | Read: ?CURR.LIM.ACT[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | 0x2314, sub-index 0 |

### Curr.Lim.User

| Parameter Name | CURR.LIM.USER |
| --- | --- |
| Definition | User current limit |
| Type | Parameter (R/W) |
| Description | Defines/reports the application current limit. This user-defined parameter value limits the current command that will be accepted from the user (CURR.CMD.USER). |
| Syntax | Write: CURR.LIM.USER[axis] = <value> Read: ?CURR.LIM.USER[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 25.455 |
| Units | A |
| CAN Index | 0x230f, sub-index 0 |

### Curr.Lpf

| Parameter Name | CURR.LPF |
| --- | --- |
| Definition | Current low pass filter |
| Type | Parameter (R/W) |
| Description | Defines/reports the current reading of the low pass filter. 0 indicates LPF is off. |
| Syntax | Write: CURR.LPF[axis] = <value> Read: ?CURR.LPF[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 8000 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x221C, sub-index 0 |

### Curr.Offset.Uint

| Parameter Name | CURR.OFFSET.UINT |
| --- | --- |
| Definition | Interpolated torque offset |
| Type | Bus (R) |
| Description | Reports the interpolated torque offset from the master. |
| Syntax | Read: ?CURR.OFFSET.UINT[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Curr.Sensor.Offset.Ia

| Parameter Name | CURR.SENSOR.OFFSET.IA |
| --- | --- |
| Definition | Phase A current sensing offset |
| Type | Parameter (R) |
| Description | Reports the A current sensing offset. |
| Syntax | Read: ?CURR.SENSOR.OFFSET.IA[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2073, sub-index 0 |

### Curr.Sensor.Offset.Ic

| Parameter Name | CURR.SENSOR.OFFSET.IC |
| --- | --- |
| Definition | Phase C current sensing offset |
| Type | Parameter (R) |
| Description | Reports the C current sensing offset. |
| Syntax | Read: ?CURR.SENSOR.OFFSET.IC[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2075, sub-index 0 |

### Curr.Sensor.Offset.Lpf

| Parameter Name | CURR.SENSOR.OFFSET.LPF |
| --- | --- |
| Definition | Current sensor offset LPF |
| Type | Parameter (R/W) |
| Description | Defines/reports the current sensor offset calculation low pass filter bandwidth [Hz]. 0 indicates LPF is off. |
| Syntax | Write: CURR.SENSOR.OFFSET.LPF[axis] = <value> Read: ?CURR.SENSOR.OFFSET.LPF[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 8000 |
| Default Value | 10 |
| Units | Hz |
| CAN Index | 0x20BF, sub-index 0 |

### Curr.Sensor.Status.A

| Parameter Name | CURR.SENSOR.STATUS.A |
| --- | --- |
| Definition | Phase A current sensor status |
| Type | Parameter (R) |
| Description | Reports the status of phase A current sensor 0 = No faults 1 = Power fault 2 = Out of range fault 3 = Power and out of range fault |
| Syntax | Read: ?CURR.SENSOR.STATUS.A[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 3 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x20C6, sub-index 0 |

### Curr.Sensor.Status.C

| Parameter Name | CURR.SENSOR.STATUS.C |
| --- | --- |
| Definition | Phase C current sensor status register |
| Type | Parameter (R) |
| Description | Reports the status of phase C current sensor 0 = No faults 1 = Power fault 2 = Out of range fault 3 = Power and out of range fault |
| Syntax | Read: ?CURR.SENSOR.STATUS.C[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 3 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23F0, sub-index 0 |

### Curr.Target.Uint

| Parameter Name | CURR.TARGET.UINT |
| --- | --- |
| Definition | Interpolated torque target |
| Type | Bus (R) |
| Description | Reports the interpolated torque target from the master. |
| Syntax | Read: ?CURR.TARGET.UINT[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Curr.Vd

| Parameter Name | CURR.VD |
| --- | --- |
| Definition | Current loop output for D direction |
| Type | Parameter (R) |
| Description | Reports the current loop output for D direction |
| Syntax | Read: ?CURR.VD[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | V |
| CAN Index | 0x2013, sub-index 0 |

### Curr.Vq

| Parameter Name | CURR.VQ |
| --- | --- |
| Definition | Current loop output for Q direction |
| Type | Parameter (R) |
| Description | Reports the current loop output for Q direction |
| Syntax | Read: ?CURR.VQ[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | V |
| CAN Index | 0x2014, sub-index 0 |

### Com.Angle (to be moved to “Commutation”)

| Parameter Name | COM.ANGLE |
| --- | --- |
| Definition | Commutation angle |
| Type | Bus (R) |
| Description | Reports the commutation angle with corrections (e.g speed corection) Need explanation? – Does this belong to Curr parameters? |
| Syntax | Read: ?COM.ANGLE[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | rad |
| CAN Index | NA |

### Com.Angle.Raw (to be moved to “Commutation”)

| Parameter Name | COM.ANGLE.RAW |
| --- | --- |
| Definition | Raw Commutation Angle |
| Type | Bus (R) |
| Description | Reports the commutation angle calculated from the position feedback. Need explanation? – Does this belong to Curr parameters? |
| Syntax | Read: ?COM.ANGLE.RAW[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | rad |
| CAN Index | NA, |

### Eso.Dcomp.Inuse (to be moved to “Commutation”)

| Parameter Name | ESO.DCOMP.INUSE |
| --- | --- |
| Definition | ESO compensation in use |
| Type | Parameter (R/W) |
| Description | Defines/reports whether the ESO is using compensation. 0 = Not in use 1 = In use. |
|  | extended state observer (ESO) |
| Syntax | Write: ESO.DCOMP.INUSE[axis] = <value> Read: ?ESO.DCOMP.INUSE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2146, sub-index 0 |

### Eso.Iff

| Parameter Name | ESO.IFF |
| --- | --- |
| Definition | Current feedforward from ESO |
| Type | Bus (R) |
| Description | Reports the current feedforward from ESO |
| Syntax | Read: ?ESO.IFF[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Ia

| Parameter Name | IA |
| --- | --- |
| Definition | Measured current on phase A |
| Type | Bus (R) |
| Description | Reports the measured current on phase A. |
| Syntax | Read: ?IA[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Iavg

| Parameter Name | IAVG |
| --- | --- |
| Definition | Measured average current |
| Type | Bus (R) |
| Description | Reports the average measured current. |
| Syntax | Read: ?IAVG[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Ic

| Parameter Name | IC |
| --- | --- |
| Definition | Measured current on phase C |
| Type | Bus (R) |
| Description | Reports the measured current on phase C. |
| Syntax | Read: ?IC[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Icmd

| Parameter Name | ICMD |
| --- | --- |
| Definition | Actual current command |
| Type | Bus (R) |
| Description | Reports the actual current command, which is generated either directly (EtherCAT/CANopen, serial or analog reference command), or as an output of the position or velocity controller |
| Syntax | Read: ?ICMD[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Icmd.Fweak.Base.Speed1

| Parameter Name | ICMD.FWEAK.BASE.SPEED1 |
| --- | --- |
| Definition | Field weakening base speed 1 |
| Type | Parameter (R) |
| Description | Reports the calculated maximum speed for maximum torque. |
| Syntax | Read: ?ICMD.FWEAK.BASE.SPEED1[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000000 |
| Default Value | 100 |
| Units | rps |
| CAN Index | 0x20A5, sub-index 0 |

### Icmd.Fweak.Base.Speed2

| Parameter Name | ICMD.FWEAK.BASE.SPEED2 |
| --- | --- |
| Definition | Field weakening base speed 2 |
| Type | Parameter (R) |
| Description | Reports the calculated maximum speed for zero torque. |
| Syntax | Read: ?ICMD.FWEAK.BASE.SPEED2[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000000 |
| Default Value | 100 |
| Units | rps |
| CAN Index | 0x20C0, sub-index 0 |

### Icmd.Fweak.Enable

| Parameter Name | ICMD.FWEAK.ENABLE |
| --- | --- |
| Definition | Enable/disable field weakening |
| Type | Parameter (R/W) |
| Description | Enable/disable field weakening. 0 = Disable field weakening 1 = Enable field weakening |
| Syntax | Write: ICMD.FWEAK.ENABLE[axis] = <value> Read: ?ICMD.FWEAK.ENABLE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x20C1, sub-index 0 |

### Icmd.Fweak.Gain

| Parameter Name | ICMD.FWEAK.GAIN |
| --- | --- |
| Definition | Field weakening gain |
| Type | Parameter (R/W) |
| Description | Defines/reports field weakening maximum voltage limiter. ?? |
|  | Defines/reports the maximum voltage limit for field weakening. |
| Syntax | Write: ICMD.FWEAK.GAIN[axis] = <value> Read: ?ICMD.FWEAK.GAIN[axis] |
| Firmware | 3.0.1.8 |
| Range | 0.75 to 1 |
| Default Value | 0.95 |
| Units | % |
| CAN Index | 0x20C2, sub-index 0 |

### Icmd.Noise.Level

| Parameter Name | ICMD.NOISE.LEVEL |
| --- | --- |
| Definition | ICMD noise level |
| Type | Bus (R) |
| Description | Reports the ICMD noise level |
| Syntax | Read: ?ICMD.NOISE.LEVEL[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Icmd.Raw

| Parameter Name | ICMD.RAW |
| --- | --- |
| Definition | Raw Current Command |
| Type | Bus (R) |
| Description | Reports the current command as provided from velocity loop |
|  | Reports the current command derived/issued from the velocity loop. ? |
| Syntax | Read: ?ICMD.RAW[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Id

| Parameter Name | ID |
| --- | --- |
| Definition | Current D Axis |
| Type | Bus (R) |
| Description | In vector control, indicates the D current for the torque. This value is perpendicular to IQ. |
| Syntax | Read: ?ID[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Iq

| Parameter Name | IQ |
| --- | --- |
| Definition | Current Q Axis |
| Type | Bus (R) |
| Description | In vector control, indicates the Q current for the torque. This value is perpendicular to ID. |
| Syntax | Read: ?IQ[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | A |
| CAN Index | NA |

### Vel.Curr

| Parameter Name | VEL.CURR |
| --- | --- |
| Definition | Actual velocity for current loop |
| Type | Bus (R) |
| Description | Reports the actual velocity derived from the raw position feedback. |
| Syntax | Read: ?VEL.CURR[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | rps |
| CAN Index | NA, sub-index 0 |

## Digital Inputs/Outputs

### Digital.In

| Parameter Name | DIGITAL.IN |
| --- | --- |
| Definition | Digital inputs status |
| Type | Parameter (R) |
| Description | Reports the state of all digital inputs or a specific digital input. |
| Syntax | Read: ?DIGITAL.IN (all inputs) ?DIGITAL.IN[0][index] (a specific input) |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to maximum number of inputs |
| Default Value | 0 |
| Units | NA |
| CAN Index | 60FD, sub-index 0 |

### Digital.In.Debounce

| Parameter Name | DIGITAL.IN.DEBOUNCE |
| --- | --- |
| Definition | Digital inputs debounce |
| Type | Parameter (R/W) |
| Description | Defines/reports the debounce time for the digital inputs. |
| Syntax | Write: DIGITAL.IN.DEBOUNCE = <value> Read: ?DIGITAL.IN.DEBOUNCE |
| Firmware | 3.0.1.8 |
| Range | 31 to 7936 |
| Default Value | 527 |
| Units | µs |
| CAN Index | 0x2702, sub-index 0 |

### Digital.In.Map

| Parameter Name | DIGITAL.IN.MAP |
| --- | --- |
| Definition | Mapping of digital inputs |
| Type | Parameter (R) |
| Description | Reports the hardware location of digital inputs. |
| Syntax | Read: ?DIGITAL.IN.MAP |
| Firmware | 3.0.1.8 |
| Range | 0 to 255 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Digital.In.Mode

| Parameter Name | DIGITAL.IN.MODE |
| --- | --- |
| Definition | Digital input single mode |
| Type | Parameter (R/W) |
| Description | Writes/reports a value that defines the digital input mode for each of the available digital inputs. 0 = Idle <num_axis>0 = Positive limit switch <num_axis>1 = Negative limit switch <num_axis>2 = Home switch DIMA |
| Syntax | Write: DIGITAL.IN.MODE[0][index] = <value> Read: ?DIGITAL.IN.MODE[0][index] |
| Firmware | 3.0.1.8 |
| Range | 0 to <max axis number>2 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x20E0, sub-index 0 |

### Digital.In.Estop

| Parameter Name | DIGITAL.IN.ESTOP |
| --- | --- |
| Definition | Digital input emergency stop mode |
| Type | Parameter (R) |
| Description | Gets the status of ESTOP for all axis. 0 = Emergency stop mode is not active 1 = Emergency stop mode is active |
| Syntax | Read: ?DIGITAL.IN.ESTOP |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2703, sub-index 0 |

### Digital.In.Numof

| Parameter Name | DIGITAL.IN.NUMOF |
| --- | --- |
| Definition | Number of digital inputs |
| Type | Parameter (R) |
| Description | Reports the number of digital inputs. |
| Syntax | Read: ?DIGITAL.IN.NUMOF |
| Firmware | 3.0.1.8 |
| Range | 0 to 255 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2700, sub-index 0 |

### Digital.In.Polarity

| Parameter Name | DIGITAL.IN.POLARITY |
| --- | --- |
| Definition | Digital inputs polarity |
| Type | Parameter (R/W) |
| Description | Defines/reports the polarity of the digital inputs. |
| Syntax | Write: DIGITAL.IN.POLARITY = <value> (all inputs) DIGITAL.IN.POLARITY[0][index] = <value> (specific input) Read: ?DIGITAL.IN.POLARITY (all inputs) ?DIGITAL.IN.POLARITY[0][index] (specific input) |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to maximum number of inputs |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2070, sub-index 0 |

### Digital.Out

| Parameter Name | DIGITAL.OUT |
| --- | --- |
| Definition | Digital outputs status |
| Type | Parameter (R/W) |
| Description | Defines/reports the state of all digital outputs or a specific digital output. |
| Syntax | Write: DIGITAL.OUT = <value> (all inputs) DIGITAL.OUT[0][index] = <value> (specific input) Read: ?DIGITAL.OUT (all inputs) ?DIGITAL.OUT[0][index] (specific input) |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to maximum number of outputs |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x60FE, sub-index 1 |

### Digital.Out.Brake

| Parameter Name | DIGITAL.OUT.BRAKE |
| --- | --- |
| Definition | Digital output brake status |
| Type | Parameter (R) |
| Description | Reports the status of the brake (brake output?) for a specific axis. DIMA… |
| Syntax | Read: ?DIGITAL.OUT.BRAKE[0][axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Digital.Out.Map

| Parameter Name | DIGITAL.OUT.MAP |
| --- | --- |
| Definition | Mapping of digital outputs |
| Type | Parameter (R) |
| Description | Reports the hardware location of digital outputs. |
| Syntax | Read: ?DIGITAL.OUT.MAP |
| Firmware | 3.0.1.8 |
| Range | 0 to 255 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Digital.Out.Mask

| Parameter Name | DIGITAL.OUT.MASK |
| --- | --- |
| Definition | Digital Outputs Mask |
| Type | Parameter (R/W) |
| Description | Defines/reports a mask value for all digital outputs. |
| Syntax | Write: DIGITAL.OUT.MASK = <value> Read: ?DIGITAL.OUT.MASK |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 4294967295 |
| Units | NA |
| CAN Index | 0x60FE, sub-index 2 |

### Digital.Out.Mode

| Parameter Name | DIGITAL.OUT.MODE |
| --- | --- |
| Definition | Digital output single mode |
| Type | Parameter (R/W) |
| Description | Writes/reports a value that defines the condition that will activate the specified digital output. 0 = Idle 10 = Alarm for any fault |
| Syntax | Write: DIGITAL.OUT.MODE[0][index] = <value> Read: ?DIGITAL.OUT.MODE[0][index] |
| Firmware | 3.0.1.8 |
| Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x209C, sub-index 0 |

### Digital.Out.Numof

| Parameter Name | DIGITAL.OUT.NUMOF |
| --- | --- |
| Definition | Number of digital outputs |
| Type | Parameter (R) |
| Description | Reports the number of digital outputs. |
| Syntax | Read: ?DIGITAL.OUT.NUMOF |
| Firmware | 3.0.1.8 |
| Range | 0 to 255 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2701, sub-index 0 |

### Digital.Out.Polarity

| Parameter Name | DIGITAL.OUT.POLARITY |
| --- | --- |
| Definition | Digital outputs polarity |
| Type | Parameter (R/W) |
| Description | Defines/reports the polarity of the digital outputs. |
| Syntax | Write: DIGITAL.OUT.POLARITY = <value> (all inputs) DIGITAL.OUT.POLARITY[0][index] = <value> (specific input) Read: ?DIGITAL.OUT.POLARITY (all inputs) ?DIGITAL.OUT.POLARITY[0][index] (specific input) |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to maximum number of outputs please confirm |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x209B, sub-index 0 |

### Digital.Led.Color

| Parameter Name | DIGITAL.LED.COLOR |
| --- | --- |
| Definition | LED color status |
| Type | Parameter (R) |
| Description | Reports the LED color of each axis. 0 = power off 1 = Red 2 = Green 3 = Blue |
| Syntax | Read: ?DIGITAL.LED.COLOR (status of the STATE (drive) LED) ?DIGITAL.LED.COLOR[0][axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 3 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

LEDs have 3 possible colors – need to add info in the QSG

## Info

### Fb.Ecat.Status

| Parameter Name | FB.ECAT.STATUS |
| --- | --- |
| Definition | EtherCAT statusword |
| Type | Variable (R) |
| Description | Reports the current state of the FSA (finite state machine), the operation mode and manufacturer specific entities. This object is organized bit-wise. The bits have the following meaning: bit 0: Ready to switch on bit 1: Switched on bit 2: Operation enabled bit 3: Fault bit 4: Voltage enabled bit 5: Quick stop bit 6: Switch on disabled bit 7: Warning bit 8: Manufacturer specific bit 9: Remote bit 10: Target reached bit 11: Internal limit active bits 12-13: Mode specific bits 14-15: Manufacturer specific |
| Syntax | Read: ?FB.ECAT.STATUS[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 65535 |
| Default Value | NA |
| Units | NA |
| CAN Index | 6041h, sub-index 0 |

### Fpga.Ver

| Parameter Name | FPGA.VER |
| --- | --- |
| Definition | FPGA version |
| Type | Variable (R) |
| Description | Reports the firmware version of the FPGA. |
| Syntax | Read: ?FPGA.VER |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Info

| Parameter Name | INFO |
| --- | --- |
| Definition | Drive info |
| Type | Command |
| Description | Reports information about the drive. |
| Syntax | Read: INFO |
| Example | info "Drive_info": "Product_number": <info> "Serial_number": <info> "Number_of_axes": <info> "SBL_Revision": <info> "FW_version": <info> "FPGA_version": <info> "Fieldbus_type": <info> "Runtime": <info> "Control_Board": <info> "Power_Board": <info> "Axis_info": <info> "Safety_Board": <info> "Motors": "Motor_info": "Axis": <num of axis> "Peak_current": <info> "Continuous_current":<info> "Feedback_type": <info> "Feedback_ID": "<info> "Network": "MAC_Address": <info> "IP_Address": <info> "IP Mask": <info> |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Status

| Parameter Name | STATUS |
| --- | --- |
| Definition | Drive status |
| Type | Command (R) |
| Description | Returns detailed drive (axis) status: Number of axis Status Enabled/Disabled Faults/Warnings status Number of opmode PFB value Velocity actual value Total faults/warnings |
| Syntax | Read: STATUS for axis all axes? STATUS <axis> for specific axis |
| Example | -1>status 1 -1>Axis 1: Status: Disabled Opmode: -8 PFB: 0.486267 [rev] V: 1.4013e-45 [rps] Total faults: [0], Total Warnings: [0] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Stat.Time.En

| Parameter Name | STAT.TIME.EN |
| --- | --- |
| Definition | Enabled time from last power up |
| Type | Parameter (R) |
| Description | Returns enabled time from last power up drive (axis) status |
| Syntax | Read: ?STAT.TIME.EN[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | s |
| CAN Index | 0x23b0, sub-index 0 |

### Stat.Time.En.Total

| Parameter Name | STAT.TIME.EN.TOTAL |
| --- | --- |
| Definition | Total enabled time from last power up |
| Type | Parameter (R) |
| Description | Returns total enabled time from last power up drive (axis) status |
| Syntax | Read: ?STAT.TIME.EN.TOTAL[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | s |
| CAN Index | 0x23af, sub-index 0 |

### Stat.Time.On

| Parameter Name | STAT.TIME.ON |
| --- | --- |
| Definition | Power on time from last power up |
| Type | Parameter (R) |
| Description | Returns power on time from last power up drive (axis) status. |
| Syntax | Read: ?STAT.TIME.ON[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | s |
| CAN Index | 0x23ae, sub-index 0 |

### Stat.Time.On.Total

| Parameter Name | STAT.TIME.ON.TOTAL |
| --- | --- |
| Definition | Power on total time from last power up |
| Type | Parameter (R) |
| Description | Returns total power on time from last power up drive (axis) status. |
| Syntax | Read: ?STAT.TIME.ON.TOTAL[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 0 |
| Default Value | 0 |
| Units | s |
| CAN Index | 0x23ad, sub-index 0 |

### Sys.Time.Date

| Parameter Name | SYS.TIME.DATE |
| --- | --- |
| Definition | System date information |
| Type | Parameter (R) |
| Description | Reports the system’s date, in the following format: Day-Month-Year Hours-Minutes-Seconds |
| Syntax | Read: ?SYS.TIME.DATE |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Sys.Time.Day

| Parameter Name | SYS.TIME.DAY |
| --- | --- |
| Definition | System day of month |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s day of month. |
| Syntax | Write: SYS.TIME.DAY = <value> Read: ?SYS.TIME.DAY |
| Firmware | 3.0.1.8 |
| Range | 1 to 31 |
| Default Value | 15 |
| Units | day |
| CAN Index | NA |

### Sys.Time.Format

| Parameter Name | SYS.TIME.FORMAT |
| --- | --- |
| Definition | System time format |
| Type | Parameter (R/W) |
| Description | Defines/reports the display format of the system’s time and date. |
| Syntax | Write: SYS.TIME.FORMAT = <value> Read: ?SYS.TIME.FORMAT |
| Example | SYS.TIME.FORMAT = %d-%b-%Y %H:%M:%S |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value |  |
| Units | NA |
| CAN Index | NA |

### Sys.Time.Hour

| Parameter Name | SYS.TIME.HOUR |
| --- | --- |
| Definition | System time hour |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s hour. |
| Syntax | Write: SYS.TIME.HOUR = <value> Read: ?SYS.TIME.HOUR |
| Firmware | 3.0.1.8 |
| Range | 0 to 23 |
| Default Value | 0 |
| Units | hour |
| CAN Index | NA |

### Sys.Time.Min

| Parameter Name | SYS.TIME.MIN |
| --- | --- |
| Definition | System time minute |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s minute. |
| Syntax | Write: SYS.TIME.MIN = <value> Read: ?SYS.TIME.MIN |
| Firmware | 3.0.1.8 |
| Range | 0 to 59 |
| Default Value | 0 |
| Units | min |
| CAN Index | NA |

### Sys.Time.Month

| Parameter Name | SYS.TIME.MONTH |
| --- | --- |
| Definition | System time month |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s month. |
| Syntax | Write: SYS.TIME.MONTH = <value> Read: ?SYS.TIME.MONTH |
| Firmware | 3.0.1.8 |
| Range | 1 to 12 |
| Default Value | 1 |
| Units | month |
| CAN Index | NA |

### Sys.Time.Sec

| Parameter Name | SYS.TIME.SEC |
| --- | --- |
| Definition | System time seconds |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s second. |
| Syntax | Write: SYS.TIME.SEC = <value> Read: ?SYS.TIME.SEC |
| Firmware | 3.0.1.8 |
| Range | 0 to 59 |
| Default Value | 0 |
| Units | sec |
| CAN Index | NA |

### Sys.Time.Timedate

| Parameter Name | SYS.TIME.TIMEDATE |
| --- | --- |
| Definition | System time and date information |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s date and time. Indexes: 0: Year 1: Month 2: Day of month 3: Hour 4: Minute 5: Second |
| Syntax | Write: SYS.TIME.TIMEDATE[index] = <value> Read: ?SYS.TIME.TIMEDATE[index] |
| Firmware | 3.0.1.8 |
| Value Range | Depends on the index |
| Index Range | 0 to 5 |
| Default Value | NA |
| Units | NA |
| CAN Index | 0x23a3, sub-index 0 |

### Sys.Time.Year

| Parameter Name | SYS.TIME.YEAR |
| --- | --- |
| Definition | System time year |
| Type | Parameter (R/W) |
| Description | Defines/reports the system’s year. |
| Syntax | Write: SYS.TIME.YEAR = <value> Read: ?SYS.TIME.YEAR |
| Firmware | 3.0.1.8 |
| Range | 2020 to 2029 |
| Default Value | 2020 |
| Units | year |
| CAN Index | NA |

### Ver

| Parameter Name | VER |
| --- | --- |
| Definition | Drive version |
| Type | Variable (R) |
| Description | Reports the firmware version of the drive. |
| Syntax | Read: ?VER |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | 100Ah, sub-index 0 |

## Motion

NOTE (for terminology)
A motion profile provides the physical motion information (defines) and graphically depicts how the motor should behave during the movement (often in terms of position, velocity, and acceleration) and is used by the servo controller to determine what commands (voltages) to send to the motor.

Motion profile = definition (of a movement)

Motion command

Movement = execution of the motion command

QUESTION:  Parameters with PTP prefix – are they all point-to-point motions? (e.g., Jog is not PTP)

PTP motion-  robot's path not necessarily a straight line path. PTP motions are accomplished using the fastest rather than the shortest path.

Point-to-Point motion allows multiaxis system to move to a specific point. (two different motion profiles available for this type of motion: Trapezoidal and Scurve)

### Jog

| Parameter Name | JOG |
| --- | --- |
| Definition | Jog command |
| Type | Parameter (R/W) |
| Description | Executes a Jog movement. Reports JOG status. Applicable in serial position and serial velocity modes. Note: Enable the axis before JOG command. |
| Syntax | Write: JOG[axis] = <value> Read: ?JOG[axis] |
| Example | Example |
| Firmware | 3.0.1.8 |
| Range | -10000 to 10000 |
| Default Value | 16.666 |
| Units | rps |
| CAN Index | NA |

### Move

| Parameter Name | MOVE |
| --- | --- |
| Definition | Motion command |
| Type | Parameter (R/W) |
| Description | Executes a position movement according to the acceleration, deceleration and velocity settings that are in effect. Reports MOVE status. Applicable in serial position and serial velocity modes. Note: Enable the axis before issuing a MOVE command. |
| Syntax | Write: MOVE[axis] = <value> Read: ?MOVE[axis] |
| Example | Example |
| Firmware | 3.0.1.8 |
| Range | -9223372036854775808 to 9223372036854775807 |
| Default Value | 0 |
| Units | 2x/rev |
| CAN Index | NA |

### P2P.Abs

| Parameter Name | P2P.ABS |
| --- | --- |
| Definition | Absolute/Relative point-to-point |
| Type | Parameter (R/W) |
| Description | Defines/reports if a point-to-point motion command is absolute or relative. 1 = Absolute 0 = Relative |
| Syntax | Write: P2P.ABS[axis] = <value> Read: ?P2P.ABS[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### P2P.Acc

| Parameter Name | P2P.ACC |
| --- | --- |
| Definition | Acceleration of point-to-point motion profile |
| Type | Parameter (R/W) |
| Description | Defines/reports the acceleration value to be used in JOG and (point-to-point) MOVE commands. |
| Syntax | Write: P2P.ACC[axis] = <value> Read: ?P2P.ACC[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 1e+07 |
| Default Value | 166.666 |
| Units | rps/s |
| CAN Index | NA |

### P2P.Acmd.Raw

| Parameter Name | P2P.ACMD.RAW |
| --- | --- |
| Definition | Point-to-point motion profile raw acceleration |
| Type | Parameter (R) |
| Description | Reports the PTP acceleration command value before the low pass filter is applied. |
| Syntax | Read: ?P2P.ACMD.RAW[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | rps/s |
| CAN Index | NA |

### P2P.Cyclic

| Parameter Name | P2P.CYCLIC |
| --- | --- |
| Definition | Cyclic motion |
| Type | Parameter (R/W) |
| Description | Defines/reports cyclic motion in serial position and serial velocity modes. 0 = No cyclic motion 1 = Cyclic motion |
| Syntax | Write: P2P.CYCLIC[axis] = <value> Read: ?P2P.CYCLIC[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### P2P.Dec

| Parameter Name | P2P.DEC |
| --- | --- |
| Definition | Deceleration of point-to-point motion profile |
| Type | Parameter (R/W) |
| Description | Defines/reports the deceleration value to be used in JOG and (point-to-point) MOVE commands. |
| Syntax | Write: P2P.DEC[axis] = <value> Read: ?P2P.DEC[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 1e+07 |
| Default Value | 166.666 |
| Units | rps/s |
| CAN Index | NA |

### P2P.Decstop

| Parameter Name | P2P.DECSTOP |
| --- | --- |
| Definition | Active Disable deceleration |
| Type | Parameter (R/W) |
| Description | Defines/reports the deceleration value for an Active Disable or Emergency Stop. |
| Syntax | Write: P2P.DECSTOP[axis] = <value> Read: ?P2P.DECSTOP[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 1e+07 |
| Default Value | 333.3333 |
| Units | rps/s |
| CAN Index | NA |

### P2P.Move.Filter

| Parameter Name | P2P.MOVE.FILTER |
| --- | --- |
| Definition | Point-to-point motion profile low pass filter |
| Type | Parameter (R/W) |
| Description | Defines/reports the Low Pass filter frequency cut-off value for the point-to-point motion profile. This filter is applied to position, velocity and acceleration commands. |
| Syntax | Write: P2P.MOVE.FILTER[axis] = <value> Read: ?P2P.MOVE.FILTER[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 4000 |
| Default Value | 4000 |
| Units | Hz |
| CAN Index | NA |

### P2P.Pcmd.Raw

| Parameter Name | P2P.PCMD.RAW |
| --- | --- |
| Definition | Point-to-point motion profile raw position |
| Type | Parameter (R) |
| Description | Reports the PTP position command value before the low pass filter is applied. |
| Syntax | Read: ?P2P.PCMD.RAW[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | 2X/rev |
| CAN Index | NA |

### P2P.Pos.Neg.Lim

| Parameter Name | P2P.POS.NEG.LIM |
| --- | --- |
| Definition | Limit negative position commands |
| Type | Parameter (R/W) |
| Description | Defines/reports whether a limit (restriction) is applied to negative position commands. 0 = Negative limitation is not active 1 = Negative limitation is active 0 = Negative position commands allowed 1 = Negative position commands not allowed |
| Syntax | Write: P2P.POS.NEG.LIM[axis] = <value> Read: ?P2P.POS.NEG.LIM[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | 2X/rev |
| CAN Index | NA |

### P2P.Pos.Pos.Lim

| Parameter Name | P2P.POS.POS.LIM |
| --- | --- |
| Definition | Limit positive position commands |
| Type | Parameter (R/W) |
| Description | Defines/reports whether a limit (restriction) is applied to positive position commands 0 = Positive limitation is not active 1 = Positive limitation is active 0 = Positive position commands allowed 1 = Positive position commands not allowed |
| Syntax | Write: P2P.POS.POS.LIM[axis] = <value> Read: ?P2P.POS.POS.LIM[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | 2X/rev |
| CAN Index | NA |

### P2P.Profile.Complete

| Parameter Name | P2P.PROFILE.COMPLETE |
| --- | --- |
| Definition | Point-to-point movement completion |
| Type | Parameter (R) |
| Description | Reports whether a point-to-point movement has been completed. 0 = PTP movement not completed 1 = PTP movement completed |
| Syntax | Read: ?P2P.PROFILE.COMPLETE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### P2P.Ptpvcmd.Raw

| Parameter Name | P2P.PTPVCMD.RAW |
| --- | --- |
| Definition | Point-to-point motion profile raw velocity |
| Type | Parameter (R) |
| Description | Reports PTP velocity command value before the low pass filter is applied. |
| Syntax | Read: ?P2P.PTPVCMD.RAW[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | rps |
| CAN Index | NA |

### P2P.Vcruise

| Parameter Name | P2P.VCRUISE |
| --- | --- |
| Definition | Velocity of the (point-to-point) motion profile. |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity value to be used in serial position and serial velocity modes. |
| Syntax | Write: P2P.VCRUISE[axis] = <value> Read: ?P2P.VCRUISE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 10000 |
| Default Value | 16.666 |
| Units | rps |
| CAN Index | NA |

### Step.Acc

| Parameter Name | STEP.ACC |
| --- | --- |
| Definition | Acceleration of step motion profile |
| Type | Parameter (R/W) |
| Description | Defines/reports the acceleration of a step motion profile. |
| Syntax | Write: STEP.ACC[axis] = <value> Read: ?STEP.ACC[axis] |
| Firmware | 3.0.1.8 |
|  | 1 to 1e+07 |
| Default Value | 166.666 |
| Units | rps/s |
| CAN Index | NA |

### Step.Dec

| Parameter Name | STEP.DEC |
| --- | --- |
| Definition | Deceleration of step motion profile |
| Type | Parameter (R/W) |
| Description | Defines/reports the deceleration of a step motion profile. |
| Syntax | Write: STEP.DEC[axis] = <value> Read: ?STEP.DEC[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 1e+07 |
| Default Value | 166.666 |
| Units | rps/s |
| CAN Index | NA |

### Step.Delay1

| Parameter Name | STEP.DELAY1 |
| --- | --- |
| Definition | Delay time after first step |
| Type | Parameter (R/W) |
| Description | Defines/reports the duration of the delay at the end of the first step motion. |
| Syntax | Write: STEP.DELAY1[axis] = <value> Read: ?STEP.DELAY1[axis] |
| Example |  |
| Firmware | 3.0.1.8 |
| Range | 0 to 2147483647 |
| Default Value | 0 |
| Units | ms |
| CAN Index | NA |

### Step.Delay2

| Parameter Name | STEP.DELAY2 |
| --- | --- |
| Definition | Delay time after second step |
| Type | Parameter (R/W) |
| Description | Defines/reports the duration of the delay at the end of the second step motion. |
| Syntax | Write: STEP.DELAY2[axis] = <value> Read: ?STEP.DELAY2[axis] |
| Example |  |
| Firmware | 3.0.1.8 |
| Range | 0 to 2147483647 |
| Default Value | 0 |
| Units | ms |
| CAN Index | NA |

### Step.Jog1

| Parameter Name | STEP.JOG1 |
| --- | --- |
| Definition | Target velocity of first step |
| Type | Parameter (R/W) |
| Description | Defines/reports the cruise speed for first sub-profile of the step. |
| Syntax | Write: STEP.JOG1[axis] = <value> Read: ?STEP.JOG1[axis] |
| Firmware | 3.0.1.8 |
| Range | -10000 to 10000 |
| Default Value | 16.666 |
| Units | rps |
| CAN Index | NA |

### Step.Jog2

| Parameter Name | STEP.JOG2 |
| --- | --- |
| Definition | Target velocity of second step |
| Type | Parameter (R/W) |
| Description | Defines/reports the cruise speed for second sub-profile of the step. |
| Syntax | Write: STEP.JOG2[axis] = <value> Read: ?STEP.JOG2[axis] |
| Firmware | 3.0.1.8 |
| Range | -10000 to 10000 |
| Default Value | -16.666 |
| Units | rps |
| CAN Index | NA |

### Step.On

| Parameter Name | STEP.ON |
| --- | --- |
| Definition | Step motion profile on/off |
| Type | Parameter (R/W) |
| Description | Defines/reports activation of the step motion profile. 0 = Step motion profile is not active 1 = Step motion profile is active |
| Syntax | Write: STEP.ON[axis] = <value> Read: ?STEP.ON[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Step.Time1

| Parameter Name | STEP.TIME1 |
| --- | --- |
| Definition | Duration of the first step motion |
| Type | Parameter (R/W) |
| Description | Defines/reports the duration of the first step motion. |
| Syntax | Write: STEP.TIME1[axis] = <value> Read: ?STEP.TIME1[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 100000 |
| Default Value | 1000 |
| Units | ms |
| CAN Index | NA |

### Step.Time2

| Parameter Name | STEP.TIME2 |
| --- | --- |
| Definition | Duration of the second step motion |
| Type | Parameter (R/W) |
| Description | Defines/reports the duration of the second step motion. |
| Syntax | Write: STEP.TIME2[axis] = <value> Read: ?STEP.TIME2[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 100000 |
| Default Value | 1000 |
| Units | ms |
| CAN Index | NA |

## Motor

### Motor.Dir

| Parameter Name | MOTOR.DIR |
| --- | --- |
| Definition | Motor direction |
| Type | Parameter (R/W) |
| Description | Defines/reports whether position feedback counts are inverted. 0 = Feedback counts are not inverted 1 = Feedback counts are inverted |
| Syntax | Write: MOTOR.DIR[axis] = <value> Read: ?MOTOR.DIR[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Motor.Fric.Dynamic

| Parameter Name | MOTOR.FRIC.DYNAMIC |
| --- | --- |
| Definition | Motor dynamic friction |
| Type | Parameter (R/W) |
| Description | Defines/reports the user-defined motor dynamic friction. Used by the Kalman Filter model. |
| Syntax | Write: MOTOR.FRIC.DYNAMIC[axis] = <value> Read: ?MOTOR.FRIC.DYNAMIC[axis] |
| Firmware | 3.0.1.8 |
| Range | 1e-06 to 50 |
| Default Value | 1e-06 |
| Units | Nm*s/rad |
| CAN Index | NA |

### Motor.Ipeak

| Parameter Name | MOTOR.IPEAK |
| --- | --- |
| Definition | Motor peak current |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor peak rated current. It is taken from the motor nameplate. |
| Syntax | Write: MOTOR.IPEAK[axis] = <value> Read: ?MOTOR.IPEAK[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | NA |

### Motor.Icont

| Parameter Name | MOTOR.ICONT |
| --- | --- |
| Definition | Motor continuous current |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor continuous rated current. It is taken from the motor nameplate. |
| Syntax | Write: MOTOR.ICONT[axis] = <value> Read: ?MOTOR.ICONT[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 8.485 |
| Units | A |
| CAN Index | NA |

### Motor.Ibrake

| Parameter Name | MOTOR.IBRAKE |
| --- | --- |
| Definition | Current for brake test |
| Type | Parameter (R/W) |
| Description | Defines/reports the current that is applied during the motor brake test mode, when the brake is engaged. |
| Syntax | Write: MOTOR.IBRAKE[axis] = <value> Read: ?MOTOR.IBRAKE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | NA |

### Motor.J

| Parameter Name | MOTOR.J |
| --- | --- |
| Definition | Motor inertia |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor rotor inertia for rotary motors. The motor rotor inertia and the load moment of inertia ratio (Motor.Lmjr) define the total system moment of inertia. |
| Syntax | Write: MOTOR.J[axis] = <value> Read: ?MOTOR.J[axis] |
| Firmware | 3.0.1.8 |
| Range | 1e-06 to 1 |
| Default Value | 1e-05 |
| Units | kg*m2 |
| CAN Index | 0x2037, sub-index 0 |

### Motor.Kt

| Parameter Name | MOTOR.KT |
| --- | --- |
| Definition | Motor torque constant |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor torque constant. This value is used for current loop controller design. |
| Syntax | Write: MOTOR.KT[axis] = <value> Read: ?MOTOR.KT[axis] |
| Firmware | 3.0.1.8 |
| Range | 0.01 to 10 |
| Default Value | 0.2 |
| Units | Nm/A |
| CAN Index | 0x2039, sub-index 0 |

### Motor.Kf

| Parameter Name | MOTOR.KF |
| --- | --- |
| Definition | Torque constant for linear motor |
| Type | Parameter (R/W) |
| Description | Defines/reports the linear motor torque constant. |
| Syntax | Write: MOTOR.KF[axis] = <value> Read: ?MOTOR.KF[axis] |
| Firmware | 3.0.1.8 |
| Range | 0.01 to 10 |
| Default Value | 0.2 |
| Units | N/A (Newton per ampere) |
| CAN Index | 0x2038, sub-index 0 |

### Motor.Lmjr

| Parameter Name | MOTOR.LMJR |
| --- | --- |
| Definition | Load to motor inertia ratio |
| Type | Parameter (R/W) |
| Description | Defines/reports the ratio of the load inertia to the motor inertia. Inertia of the motor: Jtotal = Motor.J * (1+Motor.Lmjr) The motor rotor inertia and the load moment of inertia ratio define the total system moment of inertia. |
| Syntax | Write: MOTOR.LMJR[axis] = <value> Read: ?MOTOR.LMJR[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x207a, sub-index 0 |

### Motor.L

| Parameter Name | MOTOR.L |
| --- | --- |
| Definition | Motor inductance |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor minimum line-to-line inductance. This parameter is used for current loop controller design and as an input to the vector control algorithms. |
| Syntax | Write: MOTOR.L[axis] = <value> Read: ?MOTOR.L[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 0 |
| Units | mH |
| CAN Index | 0x203a, sub-index 0 |

### Motor.Mass

| Parameter Name | MOTOR.MASS |
| --- | --- |
| Definition | Mass of linear motor without load |
| Type | Parameter (R/W) |
| Description | Defines/reports the mass of the linear motor carriage without any additional payload. This parameter is used as the basis for estimating the total moving mass. |
| Syntax | Write: MOTOR.MASS[axis] = <value> Read: ?MOTOR.MASS[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 100 |
| Default Value | 0 |
| Units | kg |
| CAN Index | 0x203d, sub-index 0 |

### Motor.Name

| Parameter Name | MOTOR.NAME |
| --- | --- |
| Definition | Motor name |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor name. |
| Syntax | Write: MOTOR.NAME[axis] = <value> Read: ?MOTOR.NAME[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Motor.Phase.Mphase

| Parameter Name | MOTOR.PHASE.MPHASE |
| --- | --- |
| Definition | Commutation phase offset |
| Type | Parameter (R/W) |
| Description | Defines/reports the communication phase offset. |
| Syntax | Write: MOTOR.PHASE.MPHASE[axis] = <value> Read: ?MOTOR.PHASE.MPHASE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 359 |
| Default Value | 0 |
| Units | deg phase |
| CAN Index | NA |

### Motor.Phase.Mvangle.Phase

| Parameter Name | MOTOR.PHASE.MVANGLE.PHASE |
| --- | --- |
| Definition | Phase advance increment |
| Type | Parameter (R/W) |
| Description | Phase advance increment (need description) |
| Syntax | Write: MOTOR.PHASE.MVANGLE.PHASE[axis] = <value> Read: ?MOTOR.PHASE.MVANGLE.PHASE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 90 |
| Default Value | 0 |
| Units | deg phase |
| CAN Index | NA |

### Motor.Phase.Mvangle.Speed

| Parameter Name | MOTOR.PHASE.MVANGLE.SPEED |
| --- | --- |
| Definition | Phase advance speed |
| Type | Parameter (R/W) |
| Description | Phase advance speed (need description) |
| Syntax | Write: MOTOR.PHASE.MVANGLE.SPEED[axis] = <value> Read: ?MOTOR.PHASE.MVANGLE.SPEED[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 10000 |
| Default Value | 0 |
| Units | rps |
| CAN Index | NA |

### Motor.Phase.Phasefind.Icmdint

| Parameter Name | MOTOR.PHASE.PHASEFIND.ICMDINT |
| --- | --- |
| Definition | Phase find process current command |
| Type | Parameter (R) |
| Description | Phase find process current command (need description) |
| Syntax | Read: ?MOTOR.PHASE.PHASEFIND.ICMDINT[axis] |
| Firmware | 3.0.1.8 |
| Range | -1000 to 1000 |
| Default Value | 0 |
| Units | A |
| CAN Index | NA |

### Motor.Phase.Phasefind.Mode

| Parameter Name | MOTOR.PHASE.PHASEFIND.MODE |
| --- | --- |
| Definition | Phase find method |
| Type | Parameter (R/W) |
| Description | Defines/reports the phase find mode. 0 = Disabled 1 = Manual 2 = Wake and shake 3 = Zeroing 4 = Wake no shake 5 = Hybrid |
| Syntax | Write: MOTOR.PHASE.PHASEFIND.MODE[axis] = <value> Read: ?MOTOR.PHASE.PHASEFIND.MODE[axis] |
| Firmware | 3.0.1.8 |
| Range | 1 to 5 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Motor.Phase.Phasefind.On

| Parameter Name | MOTOR.PHASE.PHASEFIND.ON |
| --- | --- |
| Definition | Enable phase find |
| Type | Parameter (R/W) |
| Description | Defines/reports activation of the phase find feature. 0 = Phase find is not active 1 = Phase find is active |
| Syntax | Write: MOTOR.PHASE.PHASEFIND.ON[axis] = <value> Read: ?MOTOR.PHASE.PHASEFIND.ON[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Motor.Phase.Ver

| Parameter Name | MOTOR.PHASE.VER |
| --- | --- |
| Definition | Phase advance version |
| Type | Parameter (R) |
| Description | Reports the phase advance version |
| Syntax | Read: ?MOTOR.PHASE.VER[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Motor.Phase.Zeroing.On

| Parameter Name | MOTOR.PHASE.ZEROING.ON |
| --- | --- |
| Definition | Zeroing process activation |
| Type | Parameter (R/W) |
| Description | Defines/reports activation of the zero process. 0 = Zeroing process disabled 1 = Zeroing process active |
| Syntax | Write: MOTOR.PHASE.ZEROING.ON[axis] = <value> Read: ?MOTOR.PHASE.ZEROING.ON[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Motor.Phase.Zeroing.State

| Parameter Name | MOTOR.PHASE.ZEROING.STATE |
| --- | --- |
| Definition | Zeroing process state |
| Type | Parameter (R) |
| Description | Reports the status of the zeroing process. 0 = Idle -1 to -4 = Zeroing process failed 1 to 8 = Internal process 9 = Zeroing process completed |
| Syntax | Read: ?MOTOR.PHASE.ZEROING.STATE[axis] |
| Firmware | 3.0.1.8 |
| Range | -4 to 9 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Motor.Pitch

| Parameter Name | MOTOR.PITCH |
| --- | --- |
| Definition | Motor pitch |
| Type | Parameter (R/W) |
| Description | Defines/reports the linear motor pitch |
| Syntax | Write: MOTOR.PITCH[axis] = <value> Read: ?MOTOR.PITCH[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 10000 |
| Default Value | 60 |
| Units | mm |
| CAN Index | 0x207d, sub-index 0 |

### Motor.Poles

| Parameter Name | MOTOR.POLES |
| --- | --- |
| Definition | Motor Poles |
| Type | Parameter (R/W) |
| Description | Defines/reports the number of motor poles. This variable is used for commutation control and represents the number of individual magnetic poles of the motor (not pole pairs). |
| Syntax | Write: MOTOR.POLES[axis] = <value> Read: ?MOTOR.POLES[axis] |
| Firmware | 3.0.1.8 |
| Range | 2 to 200 |
| Default Value | 2 |
| Units | NA |
| CAN Index | 0x207e, sub-index 0 |

### Motor.R

| Parameter Name | MOTOR.R |
| --- | --- |
| Definition | Motor resistence |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor resistance. |
| Syntax | Write: MOTOR.R[axis] = <value> Read: ?MOTOR.R[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 75 |
| Default Value | 0 |
| Units | Ohm |
| CAN Index | 0x207f, sub-index 0 |

### Motor.Saliency

| Parameter Name | MOTOR.SALIENCY |
| --- | --- |
| Definition | Motor saliency |
| Type | Parameter (R/W) |
| Description | Defines/reports the motor saliency. The ratio Ld/Lq (d-axis inductance and q-axis inductance): 0 = Ld/Lq = 0 1 = Ld/Lq = 1 |
| Syntax | Write: MOTOR.SALIENCY[axis] = <value> Read: ?MOTOR.SALIENCY[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Motor.Speed

| Parameter Name | MOTOR.SPEED |
| --- | --- |
| Definition | Motor Max Speed |
| Type | Parameter (R/W) |
| Description | Defines/reports the maximum velocity limit of the motor. |
| Syntax | Write: MOTOR.SPEED[axis] = <value> Read: ?MOTOR.SPEED[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 200 |
| Default Value | 0 |
| Units | rps |
| CAN Index | 0x238b, sub-index 0 |

### Motor.Type

| Parameter Name | MOTOR.TYPE |
| --- | --- |
| Definition | Motor Type |
| Type | Parameter (R/W) |
| Description | Defines/reports a value that defines the motor type. 0 = Rotary 2 = Linear |
| Syntax | Write: MOTOR.TYPE[axis] = <value> Read: ?MOTOR.TYPE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 2 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2024, sub-index 0 |

### Motor.uwSwap

| Parameter Name | MOTOR.UWSWAP |
| --- | --- |
| Definition | Phase swap between U and W phases |
| Type | Parameter (R/W) |
| Description | Defines/reports whether the current feedback and the voltage command output between the U and W phases are swapped. 0 = U and W phases are not swapped 1 = U and W phases are swapped uwSwap can be executed only only when the drive is disabled. After a uwSwap is executed, it is necessary to use zeroing to find the new Mphase. |
| Syntax | Write: MOTOR.UWSWAP[axis] = <value> Read: ?MOTOR.UWSWAP[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

## Recording

### Recdone

| Parameter Name | RECDONE |
| --- | --- |
| Definition | Recording done |
| Type | Parameter (W) |
| Description | Command to report whether the recording is finished and data is available. |
| Syntax | Write: RECDONE |
| Example | -1>recdone -1>Complete |
| Example | -1>recdone -1>Waiting for trigger, filling buffer |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Recgap

| Parameter Name | RECGAP |
| --- | --- |
| Definition | Recording sampling time |
| Type | Parameter (W) |
| Description | Defines the rate at which data is recorded. The sampling value must be equal to or greater than 1. The interval value is specified in multiples of the drive’s default sampling rate, which is 62.5 μs. For example, an interval of 4 means data is recorded once every 4 samples, or once per 250 μs (4×62.5=250), or 0.25 ms. |
| Syntax | Write: RECGAP <sample_factor> |
| Example | recoff recoffs 100 recgap 1 rectrig active[1] > 0 record 2000 active[1] en[1] = 1 #PLOT |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | multiples of 62.5 µs. |
| CAN Index | NA |

### Recoff

| Parameter Name | RECOFF |
| --- | --- |
| Definition | Stop recording |
| Type | Parameter (W) |
| Description | Command to stop an active recording. |
| Syntax | Write: RECORD |
| Example | recoff recoffs 100 recgap 1 rectrig active[1] > 0 record 6000 ptpvcmd[1] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Recoffs

| Parameter Name | RECOFFS |
| --- | --- |
| Definition | Points pre-trigger |
| Type | Parameter (W) |
| Description | Command to record a specified number of points before the trigger condition occurs. |
| Syntax | Write: RECOFFS <number_pre-points> |
| Example | recoff recoffs 100 recgap 1 rectrig active[1] > 0 record 6000 ptpvcmd[1] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | multiples of 62.5 µs. |
| CAN Index | NA |

### Record

| Parameter Name | RECORD |
| --- | --- |
| Definition | Record data |
| Type | Parameter (W) |
| Description | Command to capture real-time variables to memory. The command syntax must include the number of points to record, followed by the name/s of the parameter/s to be recorded. |
| Syntax | Write: RECORD <number_of_points> <parameter(s)> |
| Example | recoff recoffs 200 recgap 4 rectrig active[2] > 0 && active[3] == 1 ¦¦ ptpvcmd[3] > 0 record 5000 ptpvcmd[1] ptpvcmd[2] ptpvcmd[3] ptpvcmd[4] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Recordclose

| Parameter Name | RECORDCLOSE |
| --- | --- |
| Definition | Close the recording |
| Type | Parameter (W) |
| Description | Command to close and save the recorded data to the drive with a specific file name. |
| Syntax | Write: RECORDCLOSE <name_of_file> (saved in .txt format) RECORDCLOSE <name_of_file.csv> (saved in .csv format) |
| Example | recoff recoffs 1000 recgap 1 rectrig active[1] > 0 record 5000 active[1] en[1] = 1 #DELAY 2000 en[1] = 0 recordclose rec1.csv |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Rectrig

| Parameter Name | RECTRIG |
| --- | --- |
| Definition | Trigger recording |
| Type | Parameter (W) |
| Description | Command to trigger the recording. The recording mechanism tracks the value of a system variable and starts the recording as soon as the value goes higher or lower than the specified value. The recorder can use comparison and logical operators in the expression of the condition. It is also possible to include more than one condition for different axes. |
| Syntax | Write: RECTRIG <parameter + condition> |
| Example | rectrig active[1] > 0 ¦¦ active[2] > 0 && active[3] == 1 ¦¦ en[4] == 1 ¦¦ flt.exist[2] > 1 ¦¦ ptpvcmd[3] >= 10 |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | multiples of 62.5 µs. |
| CAN Index | NA |

## Temperature

### Temp.Cpu

| Parameter Name | TEMP.CPU |
| --- | --- |
| Definition | Device CPU average temperature |
| Type | Parameter (R) |
| Description | Reports the calculated average temperature of all the cores. |
| Syntax | Read: ?TEMP.CPU |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Cpu.Cor

| Parameter Name | TEMP.CPU.COR |
| --- | --- |
| Definition | Device CPU core temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the CPU core. |
| Syntax | Read: ?TEMP.CPU.COR |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Cpu.Dsp

| Parameter Name | TEMP.CPU.DSP |
| --- | --- |
| Definition | Device DSP temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the DSP. |
| Syntax | Read: ?TEMP.CPU.DSP |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Cpu.Gpu

| Parameter Name | TEMP.CPU.GPU |
| --- | --- |
| Definition | Device GPU temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the GPU. |
| Syntax | Read: ?TEMP.CPU.GPU |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Cpu.Iva

| Parameter Name | TEMP.CPU.IVA |
| --- | --- |
| Definition | Device IVA temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the IVA. |
| Syntax | Read: ?TEMP.CPU.IVA |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Cpu.Mpu

| Parameter Name | TEMP.CPU.MPU |
| --- | --- |
| Definition | Device MPU temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the MPU. |
| Syntax | Read: ?TEMP.CPU.MPU |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | NA |

### Temp.Ipm

| Parameter Name | TEMP.IPM |
| --- | --- |
| Definition | IPM temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the IPM. |
| Syntax | Read: ?TEMP.IPM[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | 0x239c, sub-index 0 |

### Temp.Motor

| Parameter Name | TEMP.MOTOR |
| --- | --- |
| Definition | Motor temperature |
| Type | Parameter (R) |
| Description | Reports the actual temperature of the motor. |
| Syntax | Read: ?TEMP.MOTOR[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 0 |
| Units | deg C |
| CAN Index | 0x239d, sub-index 0 |

### Temp.Motor.Flt.High

| Parameter Name | TEMP.MOTOR.FLT.HIGH |
| --- | --- |
| Definition | Motor over-temperature fault upper threshold |
| Type | Parameter (R/W) |
| Description | Defines/reports the upper threshold value of the motor over-temperature fault. |
| Syntax | Write: TEMP.MOTOR.FLT.HIGH[axis] = <value> Read: ?TEMP.MOTOR.FLT.HIGH[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 105 |
| Units | deg C |
| CAN Index | 0x232b, sub-index 0 |

### Temp.Motor.Flt.Low

| Parameter Name | TEMP.MOTOR.FLT.LOW |
| --- | --- |
| Definition | Motor over-temperature fault lower threshold |
| Type | Parameter (R/W) |
| Description | Defines/reports the lower threshold value of the motor over-temperature fault. |
| Syntax | Write: TEMP.MOTOR.FLT.LOW[axis] = <value> Read: ?TEMP.MOTOR.FLT.LOW[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 100 |
| Units | deg C |
| CAN Index | 0x232a, sub-index 0 |

### Temp.Motor.Wrn.High

| Parameter Name | TEMP.MOTOR.WRN.HIGH |
| --- | --- |
| Definition | Motor over-temperature warning upper threshold |
| Type | Parameter (R/W) |
| Description | Defines/reports the upper threshold value of the motor over-temperature warning. |
| Syntax | Write: TEMP.MOTOR.WRN.HIGH[axis] = <value> Read: ?TEMP.MOTOR.WRN.HIGH[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 95 |
| Units | deg C |
| CAN Index | 0x2323, sub-index 0 |

### Temp.Motor.Wrn.Low

| Parameter Name | TEMP.MOTOR.WRN.LOW |
| --- | --- |
| Definition | Motor over-temperature warning lower threshold |
| Type | Parameter (R/W) |
| Description | Defines/reports the lower threshold value of the motor over-temperature warning. |
| Syntax | Write: TEMP.MOTOR.WRN.LOW[axis] = <value> Read: ?TEMP.MOTOR.WRN.LOW[axis] |
| Firmware | 3.0.1.8 |
| Range | -60 to 250 |
| Default Value | 90 |
| Units | deg C |
| CAN Index | 0x2322, sub-index 0 |

## Variable Gains

Controller gain = sensitivity to change output due to tracking error (the difference between setpoint and feedback).

Controller tuning = finding optimum gain values for the system.

Feedback gain variables:

P-gain - reacts instantly to the tracking error but can't eliminate tracking error completely

I-gain - reacts slowly over time, adjusts output until tracking error is zero

Gain values

### Vargains.Active

| Parameter Name | VARGAINS.ACTIVE |
| --- | --- |
| Definition | Active variable gain set |
| Type | Parameter (R/W) |
| Description | Defines/reports the active variable gain set |
| Syntax | Write: VARGAINS.ACTIVE = <value> Read: ?VARGAINS.ACTIVE |
| Firmware | 3.0.1.8 |
| Range | 0 to 9 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c6, sub-index 0 |

### Vargains.Axis.Cplg

| Parameter Name | VARGAINS.AXIS.CPLG |
| --- | --- |
| Definition | Variable gains – axis coupling ratio |
| Type | Parameter (R/W) |
| Description | Defines/reports the axis coupling factor. This factor is the coupling ratio between axes. |
| Syntax | Write: VARGAINS.AXIS.CPLG[axis][index] = <value> Read: ?VARGAINS.AXIS.CPLG[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -3.402823e+38 to 3.402823e+38 |
| Index Range | 0 to 3 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c3, sub-index 0 |

### Vargains.Axis.Desc

| Parameter Name | VARGAINS.AXIS.DESC |
| --- | --- |
| Definition | Variable gains – axis description |
| Type | Parameter (R/W) |
| Description | Defines/reports the description of the axis variable gain set. This parameter provides data about the axis gain set: what is a desired combination of axis states and payload required for the gain set to be selected .. such as a suitable combination of axis states, and a suitable payload for a specific gain set. |
| Syntax | Write: VARGAINS.AXIS.DESC[0][index] = <value> Read: ?VARGAINS.AXIS.DESC[0][index] |
| Example | vargains.axis.description[0][0] = {"SystemType":"AIO", "AxisName":"Axis1", "AxisState":["FullRange;-180;180"], "AxisUnits":"Deg"} vargains.axis.description[0][1] = {"SystemType":" AIO ", "AxisName":"Axis2", "AxisState":["FullRange;-180;180"], "AxisUnits":"Deg"} vargains.axis.description[0][2] = {"SystemType":" AIO ", "AxisName":"Axis3", "AxisState":["FullRange;-170;40"], "AxisUnits":"mm"} vargains.axis.description[0][3] = {"SystemType":" AIO ", "AxisName":"Axis4", "AxisState":["FullRange;-360;360"], "AxisUnits":"Deg" |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | 0 to 3 |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Vargains.Axis.Offset

| Parameter Name | VARGAINS.AXIS.OFFSET |
| --- | --- |
| Definition | Variable gains – axis offset |
| Type | Parameter (R/W) |
| Description | Defines/reports the axis position offset factor to/from the motion controller. |
| Syntax | Write: VARGAINS.AXIS.OFFSET[axis] = <value> Read: ?VARGAINS.AXIS.OFFSET[axis] |
| Firmware | 3.0.1.8 |
| Range | -3.402823e+38 to 3.402823e+38 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c1, sub-index 0 |

### Vargains.Axis.Posfactor

| Parameter Name | VARGAINS.AXIS.POSFACTOR |
| --- | --- |
| Definition | Variable gains – axis position coefficient |
| Type | Parameter (R/W) |
| Description | Defines/reports the axis position offset factor. This coefficient includes motion controller units factors: axis position factor, axis direction, gear and feed ratio. |
| Syntax | Write: VARGAINS.AXIS.POSFACTOR[axis] = <value> Read: ?VARGAINS.AXIS.POSFACTOR[axis] |
| Firmware | 3.0.1.8 |
| Range | -3.402823e+38 to 3.402823e+38 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x23c2, sub-index 0 |

### Vargains.Axis.Status

| Parameter Name | VARGAINS.AXIS.STATUS |
| --- | --- |
| Definition | Variable gain – axis status |
| Type | Parameter (R) |
| Description | Reports the variable gain axis status. |
| Syntax | Read: ?VARGAINS.AXIS.STATUS |
| Firmware | 3.0.1.8 |
| Range | 0 to 130 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Change.Info

| Parameter Name | VARGAINS.CHANGE.INFO |
| --- | --- |
| Definition | Variable gains – change info |
| Type | Parameter (R) |
| Description | Reports changes in the variable gain: Common position error delay, Status and Status counter |
| Syntax | Read: ?VARGAINS.CHANGE.INFO |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c7, sub-index 0 |

### Vargains.Enable

| Parameter Name | VARGAINS.ENABLE |
| --- | --- |
| Definition | Variable gains – activation |
| Type | Parameter (R/W) |
| Description | Defines/reports whether the variable gain set feature is active. 0 = Variable gain set feature is not active 1 = Variable gain set feature is active |
| Syntax | Write: VARGAINS.ENABLE = <value> Read: ?VARGAINS.ENABLE |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Execute

| Parameter Name | VARGAINS.EXECUTE |
| --- | --- |
| Definition | Variable gains – execution |
| Type | Parameter (R/W) |
| Description | Defines/reports execution of a change in the variable gain set. This parameter initiates a parsing of axis and gain set descriptions. |
| Syntax | Write: VARGAINS.EXECUTE = <value> Read: ?VARGAINS.EXECUTE |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Gainset.Active

| Parameter Name | VARGAINS.GAINSET.ACTIVE |
| --- | --- |
| Definition | Variable gains – active gain set |
| Type | Parameter (R/W) |
| Description | Defines/reports the gains set number for the axis. |
| Syntax | Write: VARGAINS.GAINSET.ACTIVE[axis] = <value> Read: ?VARGAINS.GAINSET.ACTIVE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 9 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Gainset.Desc

| Parameter Name | VARGAINS.GAINSET.DESC |
| --- | --- |
| Definition | Variable gains – gain set description |
| Type | Parameter (R/W) |
| Description | Defines/reports the gain set description. This parameter provides data about the gain set: what is a desired combination of axis states and payload required for the gain set to be selected. .. such as a suitable combination of axis states, and a suitable payload for a specific gain set. |
| Syntax | Write: VARGAINS.GAINSET.DESC[0][index] = <value> Read: ?VARGAINS.GAINSET.DESC[0][index] |
| Example | vargains.gainset.desc[0][0] = {"Axis1":"FullRange", "Axis2":"FullRange", "Axis3":"FullRange", "Axis4":"FullRange", "Payload":"0.0"} vargains.gainset.desc[0][1] = {"Axis1":"FullRange", "Axis2":"FullRange", "Axis3":"FullRange", "Axis4":"FullRange", "Payload":"1.0"} vargains.gainset.desc[0][2] = {"Axis1":"FullRange", "Axis2":"FullRange", "Axis3":"FullRange", "Axis4":"FullRange", "Payload":"3.0"} vargains.gainset.desc[0][3] = {"Axis1":"FullRange", "Axis2":"FullRange", "Axis3":"FullRange", "Axis4":"FullRang |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | 0 to 9 |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Vargains.Gainset.Inuse

| Parameter Name | VARGAINS.GAINSET.INUSE |
| --- | --- |
| Definition | Variable gains – axis gain set in use |
| Type | Parameter (R/W) |
| Description | Defines/reports the gain set currently in use for the selected axis. |
| Syntax | Write: VARGAINS.GAINSET.INUSE[axis][index] = <value> Read: ?VARGAINS.GAINSET.INUSE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to 9 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Vargains.Gainset.Status

| Parameter Name | VARGAINS.GAINSET.STATUS |
| --- | --- |
| Definition | Variable gains – axis gain set status |
| Type | Parameter (R) |
| Description | Reports the axis status of the gain set after change. |
| Syntax | Read: ?VARGAINS.GAINSET.STATUS[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Vargains.Inuse

| Parameter Name | VARGAINS.INUSE |
| --- | --- |
| Definition | Variable gains in use |
| Type | Parameter (R/W) |
| Description | Defines/reports the array gain set currently in use. |
| Syntax | Write: VARGAINS.INUSE[0][index] = <value> Read: ?VARGAINS.INUSE[0][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to 9 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Joint.Pcmd

| Parameter Name | VARGAINS.JOINT.PCMD |
| --- | --- |
| Definition | Variable gains – joint position command |
| Type | Parameter (R) |
| Description | Reports the joint position command from the motion controller. 0 = 1 = |
| Syntax | Read: ?VARGAINS.JOINT.PCMD |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vargains.Payload

| Parameter Name | VARGAINS.PAYLOAD |
| --- | --- |
| Definition | Variable gains – payload |
| Type | Parameter (R/W) |
| Description | Defines/reports the actual payload for the gain set. |
| Syntax | Write: VARGAINS.PAYLOAD = <value> Read: ?VARGAINS.PAYLOAD |
| Firmware | 3.0.1.8 |
| Range | 0 to 1000 |
| Default Value | 0 |
| Units | kg |
| CAN Index | 0x23c0, sub-index 0 |

### Vargains.Pcmdact

| Parameter Name | VARGAINS.PCMDACT |
| --- | --- |
| Definition | Variable gains – actual position command |
| Type | Parameter (R/W) |
| Description | Defines/reports the actual coupled position command |
| Syntax | Write: VARGAINS.PCMDACT[axis] = <value> Read: ?VARGAINS.PCMDACT[axis] |
| Firmware | 3.0.1.8 |
| Range | -9223372036854775807 to 9223372036854775807 |
| Default Value | 0 |
| Units | 2^x/rev |
| CAN Index | NA |

### Vargains.Status

| Parameter Name | VARGAINS.STATUS |
| --- | --- |
| Definition | Variable gains – set ?execution status |
| Type | Parameter (R) |
| Description | Reports execution of the variable gain set. Overall gain change status: 0 = Change in process 1 = Change finished 1 bit per axis: bit 0 = Axis 1 status bit 1 = Axis 2 status bit n = Axis n+1 status |
| Syntax | Read: ?VARGAINS.STATUS |
| Firmware | 3.0.1.8 |
| Range | 0 to 7 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c4, sub-index 0 |

### Vargains.Status.Cntr

| Parameter Name | VARGAINS.STATUS.CNTR |
| --- | --- |
| Definition | Variable gains – gain set status counter |
| Type | Parameter (R) |
| Description | Reports the number of changes in the gain set status. The counter is incremented after a gain change is executed. The counter is zeroed if it reaches 32. |
| Syntax | Read: ?VARGAINS.STATUS.CNTR |
| Firmware | 3.0.1.8 |
| Range | 0 to 15 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23c5, sub-index 0 |

## Velocity Control

### Vact

| Parameter Name | VACT |
| --- | --- |
| Definition | Actual velocity |
| Type | Parameter (R) |
| Description | Gets the velocity of the motor measured by the primary feedback. |
| Syntax | Read: ?VACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | 0 |
| Default Value | NA |
| Units | rps |
| CAN Index | NA, sub-index 0 |

### Vact.Execute

| Parameter Name | VACT.EXECUTE |
| --- | --- |
| Definition | Vact module internal execute |
| Type | Parameter (R/W) |
| Description | Defines/reports execution configuration for velocity. Execution configuration required after modifying velocity actual parameters. |
|  | Defines/reports execution of configuration for velocity. Vact.Execute command is required after actual velocity parameters are modified. |
| Syntax | Write: VACT.EXECUTE[axis] = <value> Read: ?VACT.EXECUTE[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Vact.Kalman.Acqr

| Parameter Name | VACT.KALMAN.ACQR |
| --- | --- |
| Definition | Kalman filter A,C,q and r matrices data in JSON format |
| Type | Parameter (R) |
| Description | Gets the data to be sent to the GUI for Kalman filter design. |
| Syntax | Read: ?VACT.KALMAN.ACQR[axis] |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | 0x2685, sub-index 0 |

### Vact.Kalman.Design

| Parameter Name | VACT.KALMAN.DESIGN |
| --- | --- |
| Definition | Kalman filter design option |
| Type | Parameter (R/W) |
| Description What is L? | Defines/reports Kalman filter mode. 1 = User matrices and new L design 2 = Estimated LMJR matrices and new L design 3 = User matrices with existing L design 4 = Estimated LMJR with existing L design |
| Syntax | Write: VACT.KALMAN.DESIGN[axis] = <value> Read: ?VACT.KALMAN.DESIGN[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 4 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2686, sub-index 0 |

### Vact.Kalman.Design.Status

| Parameter Name | VACT.KALMAN.DESIGN.STATUS |
| --- | --- |
| Definition | Kalman filter design process status |
| Type | Parameter (R) |
| Description | Reports whether ACqr matrices were sent, and if L was set. To be read by the GUI for the design process. |
| Syntax | Read: ?VACT.KALMAN.DESIGN.STATUS[axis] |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | 0x2687, sub-index 0 |

### Vact.Kalman.Pobs

| Parameter Name | VACT.KALMAN.POBS |
| --- | --- |
| Definition | Kalman filter estimated position |
| Type | Parameter (R) |
| Description | Gets the Kalman filter estimated position. |
| Syntax | Read: ?VACT.KALMAN.POBS[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 100000 |
| Index Range | 0 |
| Default Value | 0 |
| Units | 2^x/rev |
| CAN Index | 0x268a, sub-index 0 |

### Vact.Kalman.Q

| Parameter Name | VACT.KALMAN.Q |
| --- | --- |
| Definition | Kalman filter input noise gain matrix |
| Type | Parameter (R/W) |
| Description | Defines/reports Kalman filter input noise gain marix. q*q^ is usually represented by Q for a Kalman filter |
| Syntax | Write: VACT.KALMAN.Q[axis][index] = <value> Read: ?VACT.KALMAN.Q[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 1e-06 to 100000 |
| Index Range | 0 to 2 |
| Default Value | 1e-06 |
| Units | NA |
| CAN Index | NA |

### Vact.Kalman.R

| Parameter Name | VACT.KALMAN.R |
| --- | --- |
| Definition | Kalman filter output noise gain |
| Type | Parameter (R/W) |
| Description | Defines/reports Kalman filter output noise gain marix. r*r^ is usually represented by R for a Kalman filter. |
| Syntax | Write: VACT.KALMAN.R[axis] = <value> Read: ?VACT.KALMAN.R[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 1e-06 to 100000 |
| Index Range | 0 |
| Default Value | 1e-06 |
| Units | NA |
| CAN Index | NA |

### Vact.Kpafrv.Factor

| Parameter Name | VACT.KPAFRV.FACTOR |
| --- | --- |
| Definition | Internal auxiliary velfilt subfactor for kpafrv factor |
| Type | Parameter (R/W) |
| Description | Gets the internal auxiliary velocity filter subfactor for kpafrv factor. |
| Syntax | Write: VACT.KPAFRV.FACTOR[axis] = <value> Read: ?VACT.KPAFRV.FACTOR[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 0 |
| Units | s |
| CAN Index | 0x232f, sub-index 0 |

### Vcmd

| Parameter Name | VCMD |
| --- | --- |
| Definition | Velocity command to position loop |
| Type | Parameter (R) |
| Description | Gets the velocity command of trajectory generator. |
| Syntax | Read: ?VCMD[axis] |
| Firmware | 3.0.1.8 |
| Value Range | NA |
| Index Range | 0 |
| Default Value | NA |
| Units | rps |
| CAN Index | NA, sub-index 0 |

### Vel.Act

| Parameter Name | VEL.ACT |
| --- | --- |
| Definition | Velocity feedback |
| Type | Parameter (R) |
| Description | Gets the actual velocity feedback. |
| Syntax | Read: ?VEL.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 2000 |
| Index Range | 0 |
| Default Value | 0 |
| Units | rps |
| CAN Index | 0x2331, sub-index 0 |

### Vel.Ctrl.Mode

| Parameter Name | VEL.CTRL.MODE |
| --- | --- |
| Definition | Velocity controller mode |
| Type | Parameter (R/W) |
| Description | Defines/reports mode for velocity controller. 1 = PDFF controller 2 = Standard pole placement controller Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.CTRL.MODE[axis][index] = <value> Read: ?VEL.CTRL.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 1 to 3 |
| Index Range | 0 to 10 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x20d4, sub-index 0 |

### Vel.Ctrl.Mode.Act

| Parameter Name | VEL.CTRL.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active mode |
| Type | Parameter (R) |
| Description | Gets the actual mode for velocity controller. 1 = PDFF controller 2 = Standard pole placement controller |
| Syntax | Read: ?VEL.CTRL.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 1 to 3 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x26f3, sub-index 0 |

### Vel.Err

| Parameter Name | VEL.ERR |
| --- | --- |
| Definition | Velocity controller error |
| Type | Parameter (R) |
| Description | Gets the velocity controller error. |
| Syntax | Read: ?VEL.ERR[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 0 |
| Units | rps |
| CAN Index | 0x26f6, sub-index 0 |

### Vel.Err.Max

| Parameter Name | VEL.ERR.MAX |
| --- | --- |
| Definition | Velocity controller error fault threshold |
| Type | Parameter (R/W) |
| Description | Defines/reports velocity controller error fault threshold. |
| Syntax | Write: VEL.ERR.MAX[axis] = <value> Read: ?VEL.ERR.MAX[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 2000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | rps |
| CAN Index | 0x2347, sub-index 0 |

### Vel.Execute

| Parameter Name | VEL.EXECUTE |
| --- | --- |
| Definition | Velocity controller execute configuration set |
| Type | Parameter (R/W) |
| Description | Defines/reports execution configuration for velocity controller. Vel.Execute command is required after velocity control parameters are modified. |
| Syntax | Write: VEL.EXECUTE[axis] = <value> Read: ?VEL.EXECUTE[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2345, sub-index 0 |

### Vel.Fb

| Parameter Name | VEL.FB |
| --- | --- |
| Definition | Velocity Controller feedback |
| Type | Parameter (R) |
| Description | Gets the velocity controller feedback. |
| Syntax | Read: ?VEL.FB[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 0 |
| Units | rps |
| CAN Index | 0x26f9, sub-index 0 |

### Vel.Fb.Filter

| Parameter Name | VEL.FB.FILTER |
| --- | --- |
| Definition | Velocity feedback LPF for velocity controller |
| Type | Parameter (R/W) |
| Description | Defines/reports LPF cutoff frequency. Note: Index defines the gain set. Note: When the value parameter is changed, Vact.Execute is required. |
| Syntax | Write: VEL.FB.FILTER[axis][index] = <value> Read: ?VEL.FB.FILTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 4000 |
| Index Range | 0 to 10 |
| Default Value | 600 |
| Units | Hz |
| CAN Index | 0x232d, sub-index 0 |

### Vel.Fb.Filter.Act

| Parameter Name | VEL.FB.FILTER.ACT |
| --- | --- |
| Definition | Velocity feedback actual LPF for velocity controller |
| Type | Parameter (R) |
| Description | Gets the actual velocity feedback LPF cutoff frequency for the velocity controller. |
| Syntax | Read: ?VEL.FB.FILTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 4000 |
| Index Range | 0 |
| Default Value | 600 |
| Units | Hz |
| CAN Index | 0x26e5, sub-index 0 |

### Vel.Fb.Filter.Current

| Parameter Name | VEL.FB.FILTER.CURRENT |
| --- | --- |
| Definition | Velocity feedback LPF for current controller |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity feedback LPF for the current controller |
| Syntax | Write: VEL.FB.FILTER.CURRENT[axis] = <value> Read: ?VEL.FB.FILTER.CURRENT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 4000 |
| Index Range | 0 |
| Default Value | 440 |
| Units | Hz |
| CAN Index | 0x26e6, sub-index 0 |

### Vel.Filt1.Lead.F

| Parameter Name | VEL.FILT1.LEAD.F |
| --- | --- |
| Definition | Velocity controller 1st filter lead frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead frequency for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.LEAD.F[axis][index] = <value> Read: ?VEL.FILT1.LEAD.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2357, sub-index 0 |

### Vel.Filt1.Lead.F.Act

| Parameter Name | VEL.FILT1.LEAD.F.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter lead frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead frequency for the first filter. |
| Syntax | Read: ?VEL.FILT1.LEAD.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2541, sub-index 0 |

### Vel.Filt1.Lead.Phase

| Parameter Name | VEL.FILT1.LEAD.PHASE |
| --- | --- |
| Definition | Velocity controller 1st filter lead phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead phase for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.LEAD.PHASE[axis][index] = <value> Read: ?VEL.FILT1.LEAD.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2358, sub-index 0 |

### Vel.Filt1.Lead.Phase.Act

| Parameter Name | VEL.FILT1.LEAD.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter lead phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead phase for the first filter. |
| Syntax | Read: ?VEL.FILT1.LEAD.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2543, sub-index 0 |

### Vel.Filt1.Mode

| Parameter Name | VEL.FILT1.MODE |
| --- | --- |
| Definition | Velocity controller 1st filter mode |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller first filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.MODE[axis][index] = <value> Read: ?VEL.FILT1.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2050, sub-index 0 |

### Vel.Filt1.Mode.Act

| Parameter Name | VEL.FILT1.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter mode |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller first filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. |
| Syntax | Read: ?VEL.FILT1.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2545, sub-index 0 |

### Vel.Filt1.Notch.Bw

| Parameter Name | VEL.FILT1.NOTCH.BW |
| --- | --- |
| Definition | Velocity controller 1st filter notch bandwidth |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch bandwidth for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.NOTCH.BW[axis][index] = <value> Read: ?VEL.FILT1.NOTCH.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2355, sub-index 0 |

### Vel.Filt1.Notch.Bw.Act

| Parameter Name | VEL.FILT1.NOTCH.BW.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter notch bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch bandwidth for the first filter. |
| Syntax | Read: ?VEL.FILT1.NOTCH.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2547, sub-index 0 |

### Vel.Filt1.Notch.Center

| Parameter Name | VEL.FILT1.NOTCH.CENTER |
| --- | --- |
| Definition | Velocity controller 1st filter notch center |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch center frequency for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.NOTCH.CENTER[axis][index] = <value> Read: ?VEL.FILT1.NOTCH.CENTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2354, sub-index 0 |

### Vel.Filt1.Notch.Center.Act

| Parameter Name | VEL.FILT1.NOTCH.CENTER.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter notch center frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch center frequency for the first filter. |
| Syntax | Read: ?VEL.FILT1.NOTCH.CENTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2549, sub-index 0 |

### Vel.Filt1.Notch.Phase

| Parameter Name | VEL.FILT1.NOTCH.PHASE |
| --- | --- |
| Definition | Velocity controller 1st filter notch phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch phase for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.NOTCH.PHASE[axis][index] = <value> Read: ?VEL.FILT1.NOTCH.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2356, sub-index 0 |

### Vel.Filt1.Notch.Phase.Act

| Parameter Name | VEL.FILT1.NOTCH.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter notch phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller 1st filter notch phase. |
| Syntax | Read: ?VEL.FILT1.NOTCH.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x254b, sub-index 0 |

### Vel.Filt1.Pole.F

| Parameter Name | VEL.FILT1.POLE.F |
| --- | --- |
| Definition | Velocity controller 1st filter pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller pole frequency first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.POLE.F[axis][index] = <value> Read: ?VEL.FILT1.POLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x235e, sub-index 0 |

### Vel.Filt1.Pole.F.Act

| Parameter Name | VEL.FILT1.POLE.F.ACT |
| --- | --- |
| Definition | Velocity Controller Active 1st Filter Pole Frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller first filter pole frequency. |
| Syntax | Read: ?VEL.FILT1.POLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x254d, sub-index 0 |

### Vel.Filt1.User

| Parameter Name | VEL.FILT1.USER |
| --- | --- |
| Definition | Velocity controller 1st filter zero user defined |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller first filter user defined. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.USER[axis][index] = <value> Read: ?VEL.FILT1.USER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 to 6 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Filt1.Xpole.F

| Parameter Name | VEL.FILT1.XPOLE.F |
| --- | --- |
| Definition | Velocity controller 1st filter complex pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex pole frequency first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.XPOLE.F[axis][index] = <value> Read: ?VEL.FILT1.XPOLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2359, sub-index 0 |

### Vel.Filt1.Xpole.F.Act

| Parameter Name | VEL.FILT1.XPOLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter complex pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex pole frequency first filter. |
| Syntax | Read: ?VEL.FILT1.XPOLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2550, sub-index 0 |

### Vel.Filt1.Xpole.Xi

| Parameter Name | VEL.FILT1.XPOLE.XI |
| --- | --- |
| Definition | Velocity controller 1st filter complex pole xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex pole for first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.XPOLE.XI[axis][index] = <value> Read: ?VEL.FILT1.XPOLE.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | Hz |
| CAN Index | 0x235a, sub-index 0 |

### Vel.Filt1.Xpole.Xi.Act

| Parameter Name | VEL.FILT1.XPOLE.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter complex pole xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller damping factor of the complex pole for the first filter. |
| Syntax | Read: ?VEL.FILT1.XPOLE.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2552, sub-index 0 |

### Vel.Filt1.Xzero.F

| Parameter Name | VEL.FILT1.XZERO.F |
| --- | --- |
| Definition | Velocity controller 1st filter complex zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex zero frequency for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.XZERO.F[axis][index] = <value> Read: ?VEL.FILT1.XZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x235b, sub-index 0 |

### Vel.Filt1.Xzero.F.Act

| Parameter Name | VEL.FILT1.XZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter complex zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex zero frequency for the first filter. |
| Syntax | Read: ?VEL.FILT1.XZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2554, sub-index 0 |

### Vel.Filt1.Xzero.Xi

| Parameter Name | VEL.FILT1.XZERO.XI |
| --- | --- |
| Definition | Velocity controller 1st filter complex zero xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex zero for first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.XZERO.XI[axis][index] = <value> Read: ?VEL.FILT1.XZERO.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x235c, sub-index 0 |

### Vel.Filt1.Xzero.Xi.Act

| Parameter Name | VEL.FILT1.XZERO.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter complex zero xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex zero Xi for the first filter. |
| Syntax | Read: ?VEL.FILT1.XZERO.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2556, sub-index 0 |

### Vel.Filt1.Zero.F

| Parameter Name | VEL.FILT1.ZERO.F |
| --- | --- |
| Definition | Velocity controller 1st filter zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller zero frequency for the first filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT1.ZERO.F[axis][index] = <value> Read: ?VEL.FILT1.ZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x235d, sub-index 0 |

### Vel.Filt1.Zero.F.Act

| Parameter Name | VEL.FILT1.ZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 1st filter zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller first filter zero frequency. |
| Syntax | Read: ?VEL.FILT1.ZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2558, sub-index 0 |

### Vel.Filt2.Lead.F

| Parameter Name | VEL.FILT2.LEAD.F |
| --- | --- |
| Definition | Velocity controller 2nd filter lead frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead frequency for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.LEAD.F[axis][index] = <value> Read: ?VEL.FILT2.LEAD.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2363, sub-index 0 |

### Vel.Filt2.Lead.F.Act

| Parameter Name | VEL.FILT2.LEAD.F.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter lead frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead frequency for the second filter |
| Syntax | Read: ?VEL.FILT2.LEAD.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x255a, sub-index 0 |

### Vel.Filt2.Lead.Phase

| Parameter Name | VEL.FILT2.LEAD.PHASE |
| --- | --- |
| Definition | Velocity controller 2nd filter lead phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead phase for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.LEAD.PHASE[axis][index] = <value> Read: ?VEL.FILT2.LEAD.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2364, sub-index 0 |

### Vel.Filt2.Lead.Phase.Act

| Parameter Name | VEL.FILT2.LEAD.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter lead phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead phase for the second filter. |
| Syntax | Read: ?VEL.FILT2.LEAD.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x255c, sub-index 0 |

### Vel.Filt2.Mode

| Parameter Name | VEL.FILT2.MODE |
| --- | --- |
| Definition | Velocity controller 2nd filter mode |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller second filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.MODE[axis][index] = <value> Read: ?VEL.FILT2.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x235f, sub-index 0 |

### Vel.Filt2.Mode.Act

| Parameter Name | VEL.FILT2.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter mode |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller second mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. |
| Syntax | Read: ?VEL.FILT2.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x255e, sub-index 0 |

### Vel.Filt2.Notch.Bw

| Parameter Name | VEL.FILT2.NOTCH.BW |
| --- | --- |
| Definition | Velocity controller 2nd filter notch bandwidth |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch bandwidth for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.NOTCH.BW[axis][index] = <value> Read: ?VEL.FILT2.NOTCH.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2361, sub-index 0 |

### Vel.Filt2.Notch.Bw.Act

| Parameter Name | VEL.FILT2.NOTCH.BW.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter notch bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch bandwidth for the second filter. |
| Syntax | Read: ?VEL.FILT2.NOTCH.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2560, sub-index 0 |

### Vel.Filt2.Notch.Center

| Parameter Name | VEL.FILT2.NOTCH.CENTER |
| --- | --- |
| Definition | Velocity controller 2nd filter notch center |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch center frequency for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.NOTCH.CENTER[axis][index] = <value> Read: ?VEL.FILT2.NOTCH.CENTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2360, sub-index 0 |

### Vel.Filt2.Notch.Center.Act

| Parameter Name | VEL.FILT2.NOTCH.CENTER.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter notch center frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch center frequency for the second filter. |
| Syntax | Read: ?VEL.FILT2.NOTCH.CENTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2562, sub-index 0 |

### Vel.Filt2.Notch.Phase

| Parameter Name | VEL.FILT2.NOTCH.PHASE |
| --- | --- |
| Definition | Velocity controller 2nd filter notch phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch phase for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.NOTCH.PHASE[axis][index] = <value> Read: ?VEL.FILT2.NOTCH.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2362, sub-index 0 |

### Vel.Filt2.Notch.Phase.Act

| Parameter Name | VEL.FILT2.NOTCH.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter notch phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller second filter notch phase. |
| Syntax | Read: ?VEL.FILT2.NOTCH.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2564, sub-index 0 |

### Vel.Filt2.Pole.F

| Parameter Name | VEL.FILT2.POLE.F |
| --- | --- |
| Definition | Velocity controller 2nd filter pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller pole frequency second filter. Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.POLE.F[axis][index] = <value> Read: ?VEL.FILT2.POLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x236a, sub-index 0 |

### Vel.Filt2.Pole.F.Act

| Parameter Name | VEL.FILT2.POLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller second filter pole frequency. |
| Syntax | Read: ?VEL.FILT2.POLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2566, sub-index 0 |

### Vel.Filt2.User

| Parameter Name | VEL.FILT2.USER |
| --- | --- |
| Definition | Velocity controller 2nd filter zero user defined |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller second filter user defined. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.USER[axis][index] = <value> Read: ?VEL.FILT2.USER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 to 6 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Filt2.Xpole.F

| Parameter Name | VEL.FILT2.XPOLE.F |
| --- | --- |
| Definition | Velocity controller 2nd filter complex pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex pole frequency second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.XPOLE.F[axis][index] = <value> Read: ?VEL.FILT2.XPOLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2365, sub-index 0 |

### Vel.Filt2.Xpole.F.Act

| Parameter Name | VEL.FILT2.XPOLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter complex pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex pole frequency second filter. |
| Syntax | Read: ?VEL.FILT2.XPOLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2569, sub-index 0 |

### Vel.Filt2.Xpole.Xi

| Parameter Name | VEL.FILT2.XPOLE.XI |
| --- | --- |
| Definition | Velocity controller 2nd filter complex pole xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex pole for second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.XPOLE.XI[axis][index] = <value> Read: ?VEL.FILT2.XPOLE.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2366, sub-index 0 |

### Vel.Filt2.Xpole.Xi.Act

| Parameter Name | VEL.FILT2.XPOLE.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter complex pole xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller damping factor of the complex pole for the second filter. |
| Syntax | Read: ?VEL.FILT2.XPOLE.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x256b, sub-index 0 |

### Vel.Filt2.Xzero.F

| Parameter Name | VEL.FILT2.XZERO.F |
| --- | --- |
| Definition | Velocity controller 2nd filter complex zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex zero frequency for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.XZERO.F[axis][index] = <value> Read: ?VEL.FILT2.XZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2367, sub-index 0 |

### Vel.Filt2.Xzero.F.Act

| Parameter Name | VEL.FILT2.XZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter complex zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex zero frequency for the second filter. |
| Syntax | Read: ?VEL.FILT2.XZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x256d, sub-index 0 |

### Vel.Filt2.Xzero.Xi

| Parameter Name | VEL.FILT2.XZERO.XI |
| --- | --- |
| Definition | Velocity controller 2nd filter complex zero xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex zero for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.XZERO.XI[axis][index] = <value> Read: ?VEL.FILT2.XZERO.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2368, sub-index 0 |

### Vel.Filt2.Xzero.Xi.Act

| Parameter Name | VEL.FILT2.XZERO.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter complex zero xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller second filter complex zero Xi. |
| Syntax | Read: ?VEL.FILT2.XZERO.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x256f, sub-index 0 |

### Vel.Filt2.Zero.F

| Parameter Name | VEL.FILT2.ZERO.F |
| --- | --- |
| Definition | Velocity controller 2nd filter zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller zero frequency for the second filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT2.ZERO.F[axis][index] = <value> Read: ?VEL.FILT2.ZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2369, sub-index 0 |

### Vel.Filt2.Zero.F.Act

| Parameter Name | VEL.FILT2.ZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 2nd filter zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller second filter zero frequency. |
| Syntax | Read: ?VEL.FILT2.ZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2571, sub-index 0 |

### Vel.Filt3.Lead.F

| Parameter Name | VEL.FILT3.LEAD.F |
| --- | --- |
| Definition | Velocity controller 3rd filter lead frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead frequency for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.LEAD.F[axis][index] = <value> Read: ?VEL.FILT3.LEAD.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2370, sub-index 0 |

### Vel.Filt3.Lead.F.Act

| Parameter Name | VEL.FILT3.LEAD.F.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter lead frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead frequency for the third filter. |
| Syntax | Read: ?VEL.FILT3.LEAD.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2573, sub-index 0 |

### Vel.Filt3.Lead.Phase

| Parameter Name | VEL.FILT3.LEAD.PHASE |
| --- | --- |
| Definition | Velocity controller 3rd filter lead phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead phase for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.LEAD.PHASE[axis][index] = <value> Read: ?VEL.FILT3.LEAD.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2371, sub-index 0 |

### Vel.Filt3.Lead.Phase.Act

| Parameter Name | VEL.FILT3.LEAD.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter lead phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead phase for the third filter. |
| Syntax | Read: ?VEL.FILT3.LEAD.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2575, sub-index 0 |

### Vel.Filt3.Mode

| Parameter Name | VEL.FILT3.MODE |
| --- | --- |
| Definition | Velocity controller 3rd filter mode |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller third filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.MODE[axis][index] = <value> Read: ?VEL.FILT3.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x236c, sub-index 0 |

### Vel.Filt3.Mode.Act

| Parameter Name | VEL.FILT3.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter mode |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller third filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. |
| Syntax | Read: ?VEL.FILT3.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2577, sub-index 0 |

### Vel.Filt3.Notch.Bw

| Parameter Name | VEL.FILT3.NOTCH.BW |
| --- | --- |
| Definition | Velocity controller 3rd filter notch bandwidth |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch bandwidth for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.NOTCH.BW[axis][index] = <value> Read: ?VEL.FILT3.NOTCH.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x236e, sub-index 0 |

### Vel.Filt3.Notch.Bw.Act

| Parameter Name | VEL.FILT3.NOTCH.BW.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter notch bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch bandwidth for the third filter. |
| Syntax | Read: ?VEL.FILT3.NOTCH.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2579, sub-index 0 |

### Vel.Filt3.Notch.Center

| Parameter Name | VEL.FILT3.NOTCH.CENTER |
| --- | --- |
| Definition | Velocity Controller 3rd Filter Notch Center |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch center frequency for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.NOTCH.CENTER[axis][index] = <value> Read: ?VEL.FILT3.NOTCH.CENTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x236d, sub-index 0 |

### Vel.Filt3.Notch.Center.Act

| Parameter Name | VEL.FILT3.NOTCH.CENTER.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter notch center frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch center frequency for the third filter. |
| Syntax | Read: ?VEL.FILT3.NOTCH.CENTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x257b, sub-index 0 |

### Vel.Filt3.Notch.Phase

| Parameter Name | VEL.FILT3.NOTCH.PHASE |
| --- | --- |
| Definition | Velocity controller 3rd filter notch phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch phase for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.NOTCH.PHASE[axis][index] = <value> Read: ?VEL.FILT3.NOTCH.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x236f, sub-index 0 |

### Vel.Filt3.Notch.Phase.Act

| Parameter Name | VEL.FILT3.NOTCH.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter notch phase |
| Type | Parameter (R) |
| Description | Velocity controller active third filter notch phase |
| Syntax | Read: ?VEL.FILT3.NOTCH.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x257d, sub-index 0 |

### Vel.Filt3.Pole.F

| Parameter Name | VEL.FILT3.POLE.F |
| --- | --- |
| Definition | Velocity controller 3rd filter pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller pole frequency third filter. Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.POLE.F[axis][index] = <value> Read: ?VEL.FILT3.POLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2377, sub-index 0 |

### Vel.Filt3.Pole.F.Act

| Parameter Name | VEL.FILT3.POLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller third filter pole frequency. |
| Syntax | Read: ?VEL.FILT3.POLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x257f, sub-index 0 |

### Vel.Filt3.User

| Parameter Name | VEL.FILT3.USER |
| --- | --- |
| Definition | Velocity controller 3rd filter user defined |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller third filter user defined. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.USER[axis][index] = <value> Read: ?VEL.FILT3.USER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 to 6 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Filt3.Xpole.F

| Parameter Name | VEL.FILT3.XPOLE.F |
| --- | --- |
| Definition | Velocity controller 3rd filter complex pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex pole frequency third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.XPOLE.F[axis][index] = <value> Read: ?VEL.FILT3.XPOLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2372, sub-index 0 |

### Vel.Filt3.Xpole.F.Act

| Parameter Name | VEL.FILT3.XPOLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter complex pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex pole frequency third filter. |
| Syntax | Read: ?VEL.FILT3.XPOLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2582, sub-index 0 |

### Vel.Filt3.Xpole.Xi

| Parameter Name | VEL.FILT3.XPOLE.XI |
| --- | --- |
| Definition | Velocity controller 3rd filter complex pole xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex pole for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.XPOLE.XI[axis][index] = <value> Read: ?VEL.FILT3.XPOLE.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2373, sub-index 0 |

### Vel.Filt3.Xpole.Xi.Act

| Parameter Name | VEL.FILT3.XPOLE.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter complex pole xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller damping factor of the complex pole for the third filter. |
| Syntax | Read: ?VEL.FILT3.XPOLE.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2584, sub-index 0 |

### Vel.Filt3.Xzero.F

| Parameter Name | VEL.FILT3.XZERO.F |
| --- | --- |
| Definition | Velocity controller 3rd filter complex zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex zero frequency for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.XZERO.F[axis][index] = <value> Read: ?VEL.FILT3.XZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2374, sub-index 0 |

### Vel.Filt3.Xzero.F.Act

| Parameter Name | VEL.FILT3.XZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter complex zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex zero frequency for the third filter. |
| Syntax | Read: ?VEL.FILT3.XZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2586, sub-index 0 |

### Vel.Filt3.Xzero.Xi

| Parameter Name | VEL.FILT3.XZERO.XI |
| --- | --- |
| Definition | Velocity controller 3rd filter complex zero Xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex zero for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.XZERO.XI[axis][index] = <value> Read: ?VEL.FILT3.XZERO.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2375, sub-index 0 |

### Vel.Filt3.Xzero.Xi.Act

| Parameter Name | VEL.FILT3.XZERO.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter complex zero Xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller third filter complex zero Xi. |
| Syntax | Read: ?VEL.FILT3.XZERO.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2588, sub-index 0 |

### Vel.Filt3.Zero.F

| Parameter Name | VEL.FILT3.ZERO.F |
| --- | --- |
| Definition | Velocity controller 3rd filter zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller zero frequency for the third filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT3.ZERO.F[axis][index] = <value> Read: ?VEL.FILT3.ZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2376, sub-index 0 |

### Vel.Filt3.Zero.F.Act

| Parameter Name | VEL.FILT3.ZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 3rd filter zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller third filter zero frequency. |
| Syntax | Read: ?VEL.FILT3.ZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x258a, sub-index 0 |

### Vel.Filt4.Lead.F

| Parameter Name | VEL.FILT4.LEAD.F |
| --- | --- |
| Definition | Velocity controller 4th filter lead frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead frequency for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.LEAD.F[axis][index] = <value> Read: ?VEL.FILT4.LEAD.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x237d, sub-index 0 |

### Vel.Filt4.Lead.F.Act

| Parameter Name | VEL.FILT4.LEAD.F.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter lead frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead frequency for the fourth filter. |
| Syntax | Read: ?VEL.FILT4.LEAD.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x258c, sub-index 0 |

### Vel.Filt4.Lead.Phase

| Parameter Name | VEL.FILT4.LEAD.PHASE |
| --- | --- |
| Definition | Velocity controller 4th filter lead phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller lead phase for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.LEAD.PHASE[axis][index] = <value> Read: ?VEL.FILT4.LEAD.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x237e, sub-index 0 |

### Vel.Filt4.Lead.Phase.Act

| Parameter Name | VEL.FILT4.LEAD.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter lead phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller lead phase for the fourth filter. |
| Syntax | Read: ?VEL.FILT4.LEAD.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x258e, sub-index 0 |

### Vel.Filt4.Mode

| Parameter Name | VEL.FILT4.MODE |
| --- | --- |
| Definition | Velocity controller 4th filter mode |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller fourth filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.MODE[axis][index] = <value> Read: ?VEL.FILT4.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2379, sub-index 0 |

### Vel.Filt4.Mode.Act

| Parameter Name | VEL.FILT4.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter mode |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller fourth filter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. |
| Syntax | Read: ?VEL.FILT4.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2590, sub-index 0 |

### Vel.Filt4.Notch.Bw

| Parameter Name | VEL.FILT4.NOTCH.BW |
| --- | --- |
| Definition | Velocity controller 4th filter notch bandwidth |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch bandwidth for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.NOTCH.BW[axis][index] = <value> Read: ?VEL.FILT4.NOTCH.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x237b, sub-index 0 |

### Vel.Filt4.Notch.Bw.Act

| Parameter Name | VEL.FILT4.NOTCH.BW.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter notch bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch bandwidth for the fourth filter. |
| Syntax | Read: ?VEL.FILT4.NOTCH.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2592, sub-index 0 |

### Vel.Filt4.Notch.Center

| Parameter Name | VEL.FILT4.NOTCH.CENTER |
| --- | --- |
| Definition | Velocity controller 4th filter notch center |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch center frequency for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.NOTCH.CENTER[axis][index] = <value> Read: ?VEL.FILT4.NOTCH.CENTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x237a, sub-index 0 |

### Vel.Filt4.Notch.Center.Act

| Parameter Name | VEL.FILT4.NOTCH.CENTER.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter notch center frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller notch center frequency for the fourth filter. |
| Syntax | Read: ?VEL.FILT4.NOTCH.CENTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2594, sub-index 0 |

### Vel.Filt4.Notch.Phase

| Parameter Name | VEL.FILT4.NOTCH.PHASE |
| --- | --- |
| Definition | Velocity controller 4th filter notch phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller notch phase for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.NOTCH.PHASE[axis][index] = <value> Read: ?VEL.FILT4.NOTCH.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x237c, sub-index 0 |

### Vel.Filt4.Notch.Phase.Act

| Parameter Name | VEL.FILT4.NOTCH.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter notch phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller active fourth filter notch phase |
| Syntax | Read: ?VEL.FILT4.NOTCH.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2596, sub-index 0 |

### Vel.Filt4.Pole.F

| Parameter Name | VEL.FILT4.POLE.F |
| --- | --- |
| Definition | Velocity Controller 4th Filter Pole Frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller pole frequency fourth filter. Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.POLE.F[axis][index] = <value> Read: ?VEL.FILT4.POLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2384, sub-index 0 |

### Vel.Filt4.Pole.F.Act

| Parameter Name | VEL.FILT4.POLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller fourth filter pole frequency. |
| Syntax | Read: ?VEL.FILT4.POLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2598, sub-index 0 |

### Vel.Filt4.User

| Parameter Name | VEL.FILT4.USER |
| --- | --- |
| Definition | Velocity controller 4th filter user defined |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller fourth filter user defined. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.USER[axis][index] = <value> Read: ?VEL.FILT4.USER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 to 6 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Filt4.Xpole.F

| Parameter Name | VEL.FILT4.XPOLE.F |
| --- | --- |
| Definition | Velocity controller 4th filter complex pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex pole frequency fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.XPOLE.F[axis][index] = <value> Read: ?VEL.FILT4.XPOLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x237f, sub-index 0 |

### Vel.Filt4.Xpole.F.Act

| Parameter Name | VEL.FILT4.XPOLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter complex pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller complex pole frequency fourth filter. |
| Syntax | Read: ?VEL.FILT4.XPOLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x259b, sub-index 0 |

### Vel.Filt4.Xpole.Xi

| Parameter Name | VEL.FILT4.XPOLE.XI |
| --- | --- |
| Definition | Velocity controller 4th filter complex pole xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex pole for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.XPOLE.XI[axis][index] = <value> Read: ?VEL.FILT4.XPOLE.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2380, sub-index 0 |

### Vel.Filt4.Xpole.Xi.Act

| Parameter Name | VEL.FILT4.XPOLE.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter complex pole xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller damping factor of the complex pole for the fourth filter. |
| Syntax | Read: ?VEL.FILT4.XPOLE.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x259d, sub-index 0 |

### Vel.Filt4.Xzero.F

| Parameter Name | VEL.FILT4.XZERO.F |
| --- | --- |
| Definition | Velocity controller 4th filter complex zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller complex zero frequency for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.XZERO.F[axis][index] = <value> Read: ?VEL.FILT4.XZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2381, sub-index 0 |

### Vel.Filt4.Xzero.F.Act

| Parameter Name | VEL.FILT4.XZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter complex zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller fourth filter complex zero frequency. |
| Syntax | Read: ?VEL.FILT4.XZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x259f, sub-index 0 |

### Vel.Filt4.Xzero.Xi

| Parameter Name | VEL.FILT4.XZERO.XI |
| --- | --- |
| Definition | Velocity controller 4th filter complex zero xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller damping factor of the complex zero for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.XZERO.XI[axis][index] = <value> Read: ?VEL.FILT4.XZERO.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2382, sub-index 0 |

### Vel.Filt4.Xzero.Xi.Act

| Parameter Name | VEL.FILT4.XZERO.XI.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter complex zero xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller fourth filter complex zero Xi. |
| Syntax | Read: ?VEL.FILT4.XZERO.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x25a1, sub-index 0 |

### Vel.Filt4.Zero.F

| Parameter Name | VEL.FILT4.ZERO.F |
| --- | --- |
| Definition | Velocity controller 4th filter zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller zero frequency for the fourth filter. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.FILT4.ZERO.F[axis][index] = <value> Read: ?VEL.FILT4.ZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2383, sub-index 0 |

### Vel.Filt4.Zero.F.Act

| Parameter Name | VEL.FILT4.ZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active 4th filter zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller fourth filter zero frequency. |
| Syntax | Read: ?VEL.FILT4.ZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x25a3, sub-index 0 |

### Vel.Integrator

| Parameter Name | VEL.INTEGRATOR |
| --- | --- |
| Definition | Velocity controller integrator |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller integrator. |
| Syntax | Read: ?VEL.INTEGRATOR[axis] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 |
| Default Value | 0 |
| Units | A |
| CAN Index | 0x2105, sub-index 0 |

### Vel.Integrator.Ff

| Parameter Name | VEL.INTEGRATOR.FF |
| --- | --- |
| Definition | Velocity integrator feedforward by antinod |
| Type | Parameter (R) |
| Description | Velocity integrator feedforward by anti-nod |
| Syntax | Read: ?VEL.INTEGRATOR.FF[axis] |
| Firmware | 3.0.1.8 |
| Value Range | -100 to 100 |
| Index Range | 0 |
| Default Value | 0 |
| Units | A |
| CAN Index | 0x2116, sub-index 0 |

### Vel.Kp.Vg

| Parameter Name | VEL.KP.VG |
| --- | --- |
| Definition | Velocity Controller Variable Gain Factor |
| Type | Parameter (R/W) |
| Description | Gets the actual velocity controller gain factor. |
| Syntax | Write: VEL.KP.VG[axis] = <value> Read: ?VEL.KP.VG[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.1 to 10 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | NA |

### Vel.Lim.Act

| Parameter Name | VEL.LIM.ACT |
| --- | --- |
| Definition | Actual velocity limit |
| Type | Parameter (R) |
| Description | Gets the actual velocity limit for the application, which is the minimum of all velocity limit inputs, such as motor speed, and application velocity limit (user limit). |
| Syntax | Read: ?VEL.LIM.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 2000 |
| Index Range | 0 |
| Default Value | 0 |
| Units | rps |
| CAN Index | 0x2330, sub-index 0 |

### Vel.Neglim

| Parameter Name | VEL.NEGLIM |
| --- | --- |
| Definition | Velocity controller negative limit tripped |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller negative limit. 0 = Negative movement allowed 1 = Negative movement not allowed |
| Syntax | Write: VEL.NEGLIM[axis] = <value> Read: ?VEL.NEGLIM[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Pdff.Kfr

| Parameter Name | VEL.PDFF.KFR |
| --- | --- |
| Definition | Velocity feedforward gain PDFF controller |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity feedforward gain for the PDFF controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PDFF.KFR[axis][index] = <value> Read: ?VEL.PDFF.KFR[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 to 10 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2025, sub-index 0 |

### Vel.Pdff.Kfr.Act

| Parameter Name | VEL.PDFF.KFR.ACT |
| --- | --- |
| Definition | Velocity controller active PDFF feedforward |
| Type | Parameter (R) |
| Description | Gets the actual velocity feedforward gain for the PDFF controller. |
| Syntax | Read: ?VEL.PDFF.KFR.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x25aa, sub-index 0 |

### Vel.Pdff.Ki

| Parameter Name | VEL.PDFF.KI |
| --- | --- |
| Definition | Velocity integrator gain PDFF controller |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity integrator gain for the PDFF controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PDFF.KI[axis][index] = <value> Read: ?VEL.PDFF.KI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 10 |
| Units | Hz |
| CAN Index | 0x2026, sub-index 0 |

### Vel.Pdff.Ki.Act

| Parameter Name | VEL.PDFF.KI.ACT |
| --- | --- |
| Definition | Velocity controller active PDFF i gain |
| Type | Parameter (R) |
| Description | Gets the actual velocity integrator gain for the PDFF controller. |
| Syntax | Read: ?VEL.PDFF.KI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1000 |
| Index Range | 0 |
| Default Value | 10 |
| Units | Hz |
| CAN Index | 0x2661, sub-index 0 |

### Vel.Pdff.Kp

| Parameter Name | VEL.PDFF.KP |
| --- | --- |
| Definition | Velocity proportional gain PDFF controller |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity proportional gain for the PDFF controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PDFF.KP[axis][index] = <value> Read: ?VEL.PDFF.KP[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 0.01 |
| Units | A/rps |
| CAN Index | 0x2027, sub-index 0 |

### Vel.Pdff.Kp.Act

| Parameter Name | VEL.PDFF.KP.ACT |
| --- | --- |
| Definition | Velocity controller active PDFF P gain |
| Type | Parameter (R) |
| Description | Gets the actual velocity proportional gain for the PDFF controller. |
| Syntax | Read: ?VEL.PDFF.KP.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1000 |
| Index Range | 0 |
| Default Value | 0.01 |
| Units | A/rps |
| CAN Index | 0x2663, sub-index 0 |

### Vel.Poslim

| Parameter Name | VEL.POSLIM |
| --- | --- |
| Definition | Velocity controller positive limit |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller positive limit. 0 = Positive movement allowed 1 = Positive movement not allowed |
| Syntax | Write: VEL.POSLIM[axis] = <value> Read: ?VEL.POSLIM[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 1 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Pp.Bw

| Parameter Name | VEL.PP.BW |
| --- | --- |
| Definition | Velocity loop bandwidth for pole placement |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity control loop bandwidth for the pole placement controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PP.BW[axis][index] = <value> Read: ?VEL.PP.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 500 |
| Index Range | 0 to 10 |
| Default Value | 20 |
| Units | Hz |
| CAN Index | 0x2010, sub-index 0 |

### Vel.Pp.Bw.Act

| Parameter Name | VEL.PP.BW.ACT |
| --- | --- |
| Definition | Velocity controller active pole placement bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity control loop bandwidth for the pole placement controller. |
| Syntax | Read: ?VEL.PP.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 500 |
| Index Range | 0 |
| Default Value | 20 |
| Units | Hz |
| CAN Index | 0x2665, sub-index 0 |

### Vel.Pp.Gain

| Parameter Name | VEL.PP.GAIN |
| --- | --- |
| Definition | Velocity controller pole placement gain |
| Type | Parameter (R/W) |
| Description | Defines/reports gain for the pole placement velocity controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PP.GAIN[axis][index] = <value> Read: ?VEL.PP.GAIN[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.1 to 10 |
| Index Range | 0 to 10 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2344, sub-index 0 |

### Vel.Pp.Gain.Act

| Parameter Name | VEL.PP.GAIN.ACT |
| --- | --- |
| Definition | Velocity controller active pole placement gain |
| Type | Parameter (R) |
| Description | Gets the actual gain for the pole placement velocity controller. |
| Syntax | Read: ?VEL.PP.GAIN.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.1 to 10 |
| Index Range | 0 |
| Default Value | 1 |
| Units | NA |
| CAN Index | 0x2667, sub-index 0 |

### Vel.Pp.Tf

| Parameter Name | VEL.PP.TF |
| --- | --- |
| Definition | Velocity tracking factor for pole placement controller |
| Type | Parameter (R/W) |
| Description | Defines/reports tracking factor for the pole placement velocity controller. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PP.TF[axis][index] = <value> Read: ?VEL.PP.TF[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 200 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | NA |
| CAN Index | 0x20c3, sub-index 0 |

### Vel.Pp.Tf.Act

| Parameter Name | VEL.PP.TF.ACT |
| --- | --- |
| Definition | Velocity controller active pole placement tracking factor |
| Type | Parameter (R) |
| Description | Gets the actual tracking factor for the pole placement velocity controller. |
| Syntax | Read: ?VEL.PP.TF.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 200 |
| Index Range | 0 |
| Default Value | 100 |
| Units | NA |
| CAN Index | 0x2669, sub-index 0 |

### Vel.Prefilt.Lead.F

| Parameter Name | VEL.PREFILT.LEAD.F |
| --- | --- |
| Definition | Velocity controller prefilter lead frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter lead frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.LEAD.F[axis][index] = <value> Read: ?VEL.PREFILT.LEAD.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | NA |
| CAN Index | 0x234c, sub-index 0 |

### Vel.Prefilt.Lead.F.Act

| Parameter Name | VEL.PREFILT.LEAD.F.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter lead frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter lead frequency. |
| Syntax | Read: ?VEL.PREFILT.LEAD.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x266b, sub-index 0 |

### Vel.Prefilt.Lead.Phase

| Parameter Name | VEL.PREFILT.LEAD.PHASE |
| --- | --- |
| Definition | Velocity controller prefilter lead phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller Prefilter lead phase. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.LEAD.PHASE[axis][index] = <value> Read: ?VEL.PREFILT.LEAD.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x234d, sub-index 0 |

### Vel.Prefilt.Lead.Phase.Act

| Parameter Name | VEL.PREFILT.LEAD.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter lead phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter lead phase. |
| Syntax | Read: ?VEL.PREFILT.LEAD.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x266d, sub-index 0 |

### Vel.Prefilt.Mode

| Parameter Name | VEL.PREFILT.MODE |
| --- | --- |
| Definition | Velocity controller prefilter mode |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.MODE[axis][index] = <value> Read: ?VEL.PREFILT.MODE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 to 10 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x2348, sub-index 0 |

### Vel.Prefilt.Mode.Act

| Parameter Name | VEL.PREFILT.MODE.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter mode |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter mode. Modes: 0 = Transparent, the filter is transparent, output = input 1 = Pole 2 = Zero 3 = Notch 4 = Complex Pole 5 = Complex Zero 6 = Lead 7 = Asymmetric Notch 8 = User Note: Index defines the gain set. |
| Syntax | Read: ?VEL.PREFILT.MODE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 8 |
| Index Range | 0 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x266f, sub-index 0 |

### Vel.Prefilt.Notch.Bw

| Parameter Name | VEL.PREFILT.NOTCH.BW |
| --- | --- |
| Definition | Velocity controller prefilter notch bandwidth |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter notch center frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.NOTCH.BW[axis][index] = <value> Read: ?VEL.PREFILT.NOTCH.BW[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 to 10 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x234a, sub-index 0 |

### Vel.Prefilt.Notch.Bw.Act

| Parameter Name | VEL.PREFILT.NOTCH.BW.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter notch bandwidth |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter notch bandwidth. |
| Syntax | Read: ?VEL.PREFILT.NOTCH.BW.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 1000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | Hz |
| CAN Index | 0x2671, sub-index 0 |

### Vel.Prefilt.Notch.Center

| Parameter Name | VEL.PREFILT.NOTCH.CENTER |
| --- | --- |
| Definition | Velocity controller prefilter notch center frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter notch center frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.NOTCH.CENTER[axis][index] = <value> Read: ?VEL.PREFILT.NOTCH.CENTER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2349, sub-index 0 |

### Vel.Prefilt.Notch.Center.Act

| Parameter Name | VEL.PREFILT.NOTCH.CENTER.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter notch center frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter notch center frequency. |
| Syntax | Read: ?VEL.PREFILT.NOTCH.CENTER.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 3000 |
| Index Range | 0 |
| Default Value | 1000 |
| Units | Hz |
| CAN Index | 0x2673, sub-index 0 |

### Vel.Prefilt.Notch.Phase

| Parameter Name | VEL.PREFILT.NOTCH.PHASE |
| --- | --- |
| Definition | Velocity controller prefilter notch phase |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity prefilter controller notch. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.NOTCH.PHASE[axis][index] = <value> Read: ?VEL.PREFILT.NOTCH.PHASE[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 to 10 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x234b, sub-index 0 |

### Vel.Prefilt.Notch.Phase.Act

| Parameter Name | VEL.PREFILT.NOTCH.PHASE.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter notch phase |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter notch phase. |
| Syntax | Read: ?VEL.PREFILT.NOTCH.PHASE.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 5 to 65 |
| Index Range | 0 |
| Default Value | 50 |
| Units | deg phase |
| CAN Index | 0x2675, sub-index 0 |

### Vel.Prefilt.Pole.F

| Parameter Name | VEL.PREFILT.POLE.F |
| --- | --- |
| Definition | Velocity controller prefilter pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter pole frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.POLE.F[axis][index] = <value> Read: ?VEL.PREFILT.POLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2353, sub-index 0 |

### Vel.Prefilt.Pole.F.Act

| Parameter Name | VEL.PREFILT.POLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter pole frequency. |
| Syntax | Read: ?VEL.PREFILT.POLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2690, sub-index 0 |

### Vel.Prefilt.User

| Parameter Name | VEL.PREFILT.USER |
| --- | --- |
| Definition | Velocity controller prefilter user defined |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller user defined. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.USER[axis][index] = <value> Read: ?VEL.PREFILT.USER[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | -100000 to 100000 |
| Index Range | 0 to 6 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Vel.Prefilt.Xpole.F

| Parameter Name | VEL.PREFILT.XPOLE.F |
| --- | --- |
| Definition | Velocity controller prefilter complex pole frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter complex pole frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.XPOLE.F[axis][index] = <value> Read: ?VEL.PREFILT.XPOLE.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x234e, sub-index 0 |

### Vel.Prefilt.Xpole.F.Act

| Parameter Name | VEL.PREFILT.XPOLE.F.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter complex pole frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter complex pole frequency. |
| Syntax | Read: ?VEL.PREFILT.XPOLE.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2678, sub-index 0 |

### Vel.Prefilt.Xpole.Xi

| Parameter Name | VEL.PREFILT.XPOLE.XI |
| --- | --- |
| Definition | velocity controller prefilter complex pole Xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter complex damping factor pole. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.XPOLE.XI[axis][index] = <value> Read: ?VEL.PREFILT.XPOLE.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x234f, sub-index 0 |

### Vel.Prefilt.Xpole.Xi.Act

| Parameter Name | VEL.PREFILT.XPOLE.XI.ACT |
| --- | --- |
| Definition | velocity controller active prefilter complex pole Xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter complex damping factor pole. |
| Syntax | Read: ?VEL.PREFILT.XPOLE.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x267a, sub-index 0 |

### Vel.Prefilt.Xzero.F

| Parameter Name | VEL.PREFILT.XZERO.F |
| --- | --- |
| Definition | Velocity controller prefilter complex zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter complex zero frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.XZERO.F[axis][index] = <value> Read: ?VEL.PREFILT.XZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2350, sub-index 0 |

### Vel.Prefilt.Xzero.F.Act

| Parameter Name | VEL.PREFILT.XZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter complex zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter complex zero frequency. |
| Syntax | Read: ?VEL.PREFILT.XZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x267c, sub-index 0 |

### Vel.Prefilt.Xzero.Xi

| Parameter Name | VEL.PREFILT.XZERO.XI |
| --- | --- |
| Definition | Velocity controller prefilter complex zero xi |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter complex damping factor zero. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.XZERO.XI[axis][index] = <value> Read: ?VEL.PREFILT.XZERO.XI[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 to 10 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x2351, sub-index 0 |

### Vel.Prefilt.Xzero.Xi.Act

| Parameter Name | VEL.PREFILT.XZERO.XI.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter complex zero xi |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter complex damping factor zero. |
| Syntax | Read: ?VEL.PREFILT.XZERO.XI.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0.001 to 1 |
| Index Range | 0 |
| Default Value | 0.707 |
| Units | NA |
| CAN Index | 0x267e, sub-index 0 |

### Vel.Prefilt.Zero.F

| Parameter Name | VEL.PREFILT.ZERO.F |
| --- | --- |
| Definition | Velocity controller prefilter zero frequency |
| Type | Parameter (R/W) |
| Description | Defines/reports the velocity controller prefilter zero frequency. Note: When the value parameter is changed, Vel.Execute is required. |
| Syntax | Write: VEL.PREFILT.ZERO.F[axis][index] = <value> Read: ?VEL.PREFILT.ZERO.F[axis][index] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 to 10 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2352, sub-index 0 |

### Vel.Prefilt.Zero.F.Act

| Parameter Name | VEL.PREFILT.ZERO.F.ACT |
| --- | --- |
| Definition | Velocity controller active prefilter zero frequency |
| Type | Parameter (R) |
| Description | Gets the actual velocity controller prefilter zero frequency |
| Syntax | Read: ?VEL.PREFILT.ZERO.F.ACT[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 20 to 3000 |
| Index Range | 0 |
| Default Value | 1500 |
| Units | Hz |
| CAN Index | 0x2680, sub-index 0 |

### Vlim

| Parameter Name | VLIM |
| --- | --- |
| Definition | User velocity limit |
| Type | Parameter (R/W) |
| Description | Defines/reports the application velocity limit, allowing the user to limit the motor peak velocity. VLIM limits the velocity command that will be accepted from the user or issued by the control loops. |
| Syntax | Write: VLIM[axis] = <value> Read: ?VLIM[axis] |
| Firmware | 3.0.1.8 |
| Value Range | 0 to 2000 |
| Index Range | 0 |
| Default Value | 100 |
| Units | rps |
| CAN Index | 0x232c, sub-index 0 |

## Warnings and Faults

| Parameter Name | FLT.MUTE |
| --- | --- |
| Definition | Fault mute |
| Type | Parameter (W) |
| Description | Mutes a specific fault for an axis. |
| Syntax | Write: FLT.MUTE[axis] = <fault number> |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Action

| Parameter Name | FLT.ACTION |
| --- | --- |
| Definition | Fault response type |
| Type | Parameter (R) |
| Description | Reports the required fault response |
| Syntax | Read: ?FLT.ACTION[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 15 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Active.List

| Parameter Name | FLT.ACTIVE.LIST |
| --- | --- |
| Definition | Fault active list |
| Type | Parameter (R) |
| Description | Reports the active faults list for the axis. Read index 0 to see the number of active faults. Other indexes indicate the ID of the active fault. DIMA |
| Syntax | Read: ?FLT.ACTIVE.LIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 0 |
| Units | NA |
| CAN Index | 0x23b7, sub-index 0 |

### Flt.Can.Error.Code

| Parameter Name | FLT.CAN.ERROR.CODE |
| --- | --- |
| Definition | CAN error code |
| Type | Parameter (R) |
| Description | Returns the CAN error code of the first latched fault. |
| Syntax | Read: ?FLT.CAN.ERROR.CODE[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 65535 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Clear

| Parameter Name | FLT.CLEAR |
| --- | --- |
| Definition | Clear faults |
| Type | Parameter (W) |
| Description | Clears all faults for the specific axis. 0 = 1 = |
| Syntax | Write: FLT.CLEAR[axis] = <value> |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Unmute

| Parameter Name | FLT.UNMUTE |
| --- | --- |
| Definition | Disable fault mute |
| Type | Parameter (W) |
| Description | Disables muting of a specific fault for the axis. |
| Syntax | Write: FLT.UNMUTE[axis] = <fault number> |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Exist

| Parameter Name | FLT.EXIST |
| --- | --- |
| Definition | Fault exists |
| Type | Parameter (R) |
| Description | Reports whether a fault exists in the axis. 0 = 1 = |
| Syntax | Read: ?FLT.EXIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Flt.Hist

| Parameter Name | FLT.HIST |
| --- | --- |
| Definition | Fault history |
| Type | Parameter (R) |
| Description | Reports the fault history of the axis. Index 0 is most recent fault. |
| Syntax | Read: ?FLT.HIST[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Flt.Last

| Parameter Name | FLT.LAST |
| --- | --- |
| Definition | First latched fault |
| Type | Parameter (R/W) |
| Description | Reports the first latched fault in the axis. 0 = 1 = |
| Syntax | Write: FLT.LAST[axis] = <value> Read: ?FLT.LAST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Inhibit.Active.List

| Parameter Name | INHIBIT.ACTIVE.LIST |
| --- | --- |
| Definition | Active inhibits |
| Type | Parameter (R) |
| Description | Reports the list of active inhibits for the axis. Read index 0 to see the number of active inhibits. Other indexes indicate the IDs of active inhibits. DIMA |
| Syntax | Read: ?INHIBIT.ACTIVE.LIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Inhibit.Exist

| Parameter Name | INHIBIT.EXIST |
| --- | --- |
| Definition | Inhibit exist |
| Type | Parameter (R) |
| Description | Reports whether inhibit exists for the axis. |
| Syntax | Read: ?INHIBIT.EXIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Inhibit.Hist

| Parameter Name | INHIBIT.HIST |
| --- | --- |
| Definition | Inhibit history |
| Type | Parameter (R) |
| Description | Reports the inhibit history of the axis. Index 0 is most recent fault. |
| Syntax | Read: ?INHIBIT.HIST[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |

### Wrn.Active.List

| Parameter Name | WRN.ACTIVE.LIST |
| --- | --- |
| Definition | Warning active list |
| Type | Parameter (R) |
| Description | Reports active warnings list for the axis. Read index 0 to see the number of active warnings. Other indexes indicate the IDs of active warnings. |
| Syntax | Read: ?WRN.ACTIVE.LIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 4294967295 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Wrn.Exist

| Parameter Name | WRN.EXIST |
| --- | --- |
| Definition | Warning exist |
| Type | Parameter (R) |
| Description | Reports whether warning exists in the axis |
| Syntax | Read: ?WRN.EXIST[axis] |
| Firmware | 3.0.1.8 |
| Range | 0 to 1 |
| Default Value | 0 |
| Units | NA |
| CAN Index | NA |

### Wrn.Hist

| Parameter Name | WRN.HIST |
| --- | --- |
| Definition | Warning history |
| Type | Parameter (R) |
| Description | Reports the warning history of the axis. Index 0 is most recent warning. |
| Syntax | Read: ?WRN.HIST[axis] |
| Firmware | 3.0.1.8 |
| Range | NA |
| Default Value | NA |
| Units | NA |
| CAN Index | NA |